# Chapter 2

## Mainboard Installation

To install this mainboard into your system, follow the procedures in this chapter:

- Identify the mainboard components

- Install the correct processor

- Install one or more memory modules

- Verify that any jumpers or switches are at the correct setting

- Install the mainboard in the system chassis

- Install extension brackets/options

- Install any other devices and make the appropriate connections to the mainboard headers.

Note: Before installing the mainboard, you must ensure that jumper JP3 is set to the Normal setting. See this chapter for information on locating JP3 and changing the jumper setting.

Note: Please do not use the AC power cord to connect the system case to a power outlet until you have completely installed the mainboard and components. In some circumstances, the power management of the system might damage components and create unsafe conditions by allowing power to flow before the installation is complete,

Note: The PCI2 slot can only be used if you disable the onboard LAN network using jumper JP6.

### Mainboard Components

Use the diagram below to identify the major components on your mainboard.

Note: Any jumpers on your mainboard that do not appear in this illustration are for testing only.

### Install the Processor

This mainboard is installed with a socket-7, and so it may be installed with any of the socket-7 processors including the Intel P55C (MMX) series, the Cyrix/IBM 6x86L/6x86MX/M-II series, the AMD K6/K6-2/K6-III series, and the IDT C6/Winchip 2/2A series. The mainboard supports system bus speeds of 60, 66, 75, 83, 95, 100 MHz.

The board supports CPU plug and play, so it can automatically run the installed processor with the correct clock speed and the correct system bus frequency. To automatically configure the processor, use the BIOS setup program to select the CPU speed and system bus frequency. See chapter three for more information.

To ensure reliability, make sure that your socket-7 processor is fitted with a heatsink/cooling fan assembly.

The socket-7 processor installs into the ZIF (Zero Insertion Force) socket-7 on the mainboard.

- Locate the socket-7 and FAN1. Pull the locking lever out from the socket and swing it to the upright position.

FAN1

Socket-7

Pin-1 Corner

- On the socket-7 processor, identify the pin-1 corner by noting that it has a slight bevel.

- On the socket-7, identify the pin-1 corner. The pin-1 corner is on the same side as the locking lever, closest to the top of the lever when it is in the locked position.

- Match the pin-1 corners and insert the socket-7 processor into the socket. No force is required and the processor should drop into place freely.

- Swing the locking lever down and hook it under the catch on the side of the socket. This locks the socket-7 processor in the socket.

- If the socket-7 processor is installed with a cooling fan assembly, connect the cable from the fan to the CPU fan power connector FAN1.

### Install Memory

The mainboard has three DIMM slots that can be installed with memory modules. You must install at least one memory module in order to use the mainboard. You must install the first memory module into DIMM1 so that the system can share some of the memory with the built-in graphics system. A second module can be installed in either DIMM2 or DIMM3.

For this mainboard, you must use 168-pin, 3.3V memory modules installed with SDRAM memory chips. For best performance, we recommend that you use PC-100 memory modules which operate over a 100 MHz memory bus.

You can install any size of memory module from 16 MB up to 256 MB, so the maximum memory size is 3 x 256 MB = 768 MB.

The edge connectors on the memory modules have cut outs, which coincide with struts in the DIMM slots, so the memory modules can only be installed in the correct way.

On the DIMM slot, pull the locking latches at either end of the slots outwards. Position the memory module correctly and insert it into the DIMM slot. Press the module down into the slot so that the locking latches lever inwards and lock the module in place.

### Set the Jumpers

JP10

JP3

JP8

1

1

JP2

1

JP6

1

Jumpers are sets of pins which can be connected together with jumper caps. The jumper caps change the way the mainboard operates by changing the electronic circuits on the mainboard. If a jumper cap connects two pins, we say the pins are SHORT. If a jumper cap is removed from two pins, the pins are OPEN.

#### Jumper JP3: Clear CMOS Memory

Use this jumper to clear the contents of the CMOS memory. You may need to clear the CMOS memory if the settings in the setup utility are incorrect and prevent your mainboard from operating. To clear the CMOS memory, disconnect all the power cables from the mainboard and then move the jumper cap into the CLEAR setting for a few seconds.

| Function | Jumper Setting |
| --- | --- |
| Normal Operation | Short Pins 1-2 |
| Clear CMOS memory | Short Pins 2-3 |

Note: The mainboard ships with this jumper in the CLEAR position so you must change this jumper to NORMAL.

#### Jumper JP2: Keyboard Power On Selector

If you are using an ATX power supply unit, you can use hot keys on your keyboard as a power on/off switch for the system. Use this jumper to turn this feature on or off.

Note: Make sure that the system can provide 1A on +5VSB (+5V Standby) signal before using the Keyboard Power On function.

| Function | Jumper Setting |
| --- | --- |
| Disable Keyboard Power On | Short Pins 1-2 |
| Enable Keyboard Power On | Short Pins 2-3 |

#### Jumper JP6: LAN Enable/disable Selector

This mainboard has a built-in 10BaseT/100BaseTX network adapter. If you plan on using an alternative network adapter, you must use this jumper to disable the onboard network adapter.

| Function | Jumper Setting |
| --- | --- |
| Enable onboard LAN | Short Pins 1-2 |
| Disable onboard LAN | Short Pins 2-3 |

#### Jumper JP8: Audio System Enable/disable

Use this jumper to enable or disable the audio system integrated on this mainboard. Disable the built-in audio if you plan on using another audio system on an expansion card.

| Function | Jumper Setting |
| --- | --- |
| Enable Audio | Open Pins 1-2 |
| Disable Audio | Short Pins 1-2 |

Note: If you disable the onboard audio system, you cannot use the onboard Fax/Modem.

#### Jumper JP10: Fax/Modem Enable/disable

Use this jumper to enable or disable the onboard Fax/Modem. Disable the Fax/Modem if you plan on using another Fax/Modem.

| Function | Jumper Setting |
| --- | --- |
| Enable Fax/Modem | Short Pins 2-3 |
| Disable Fax/Modem | Short Pins 1-2 |

Note: If you have disabled the onboard audio system with jumper JP8, the Fax/Modem will not function even if it is enabled.

### Install the Mainboard

Install the mainboard into the system chassis. This mainboard uses the baby-AT format. In addition the mainboard can operate using an AT power supply unit or an ATX power supply unit. This means that you have a wide choice of cases that can be used by this mainboard.

Install the mainboard into the unit case. Follow the instructions provided by the case manufacturer using the screws and mounting points provided in the chassis.

ATX-PWR2

J2

1

AT-PWR1

Connect the power cable from the power supply unit to the power connector on the mainboard. If you are using an AT power unit, connect it to the AT power connector PWR1. If you are using an ATX power unit connect it to the ATX power connector PWR2. Connect the case switches and indicator LEDs to the bank of switch and LED connectors J2. See the illustration below for a guide to the pin functions of the J2 connector.

21 22

Power LED

Pins 2-4-6

Speaker

Pins 1-3-5-7

Keylock

Pins 8-10

HDD LED Pins 15-16

1  2

Reset Switch Pins 17-18

Power Switch Pins 21-22

### Install the Extension Brackets/Options

The modules and extension brackets are used to transmit features on the mainboard to external connectors that can be fixed to the system chassis. Follow the steps below to install the extension brackets.

Note: All the ribbon cables used on the extension brackets carry a red stripe on the pin-1 side of the cable.

#### Audio Ports and Game/MIDI Port Extension Bracket

This bracket provides three audio jacks for stereo line in, stereo line out and microphone. In addition it has a 15-pin D-connector which can be used by either a joystick or a MIDI device.

If you are using a four channel speaker system, channel one and two are output through the Stereo Line-out, and the rear speaker channels three and four are output through Stereo Line-in.

Audio Ports & Game/MIDI Extension Bracket

Stereo Line-in /Rear speaker channels 3-4

Microphone

Stereo Line-out /Speaker channels 1-2

Game/MIDI

J6-Audio Header

1

- On the mainboard, locate the J6 audio header for this bracket.

- Plug the cable from the bracket into the J6 audio header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Serial/Parallel Ports Extension Bracket

This bracket has one serial port - COM1 (9-pins) and one parallel port – LPT1 (25pins).

1

COM1 Header

PRN1- Parallel Port Header

1

Serial Port

COM1

Parallel Port

LPT1

Serial/Parallel Ports

Extension Bracket

- On the mainboard, locate the headers COM1 and PRN1 for this bracket.

- Plug the serial cable into COM1 and the parallel cable into PRN1.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### VGA Extension Bracket

The VGA extension bracket has a 15-pin connector for an external monitor cable.

VGA1-VGA Header

1

VGA Extension Bracket

- On the mainboard, locate the VGA1 header for this bracket.

- Plug the cable from the bracket into the VGA1 header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### LAN Network Adapter Extension Bracket

This bracket supports an RJ45 network connector and connects to the built in LAN header LAN1 on the mainboard.

LAN1 LAN Header

LAN Extension Bracket

1

- On the mainboard, locate the LAN1 LAN header for this bracket.

- Plug the cable from the bracket into LAN1.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Fax/Modem DAA Module

2 MCLK

4 FRA-SY

6 SCLK

8 RIN-WAK

10 GPIO

12 SDO

14 SDI

16 RST

GND 1

GND 3

GND 5

AUX 3V 7

HOOK 9

RIN 11

AUX 5V 13

MUTE 15

J7-MODEM Header

The Fax/Modem DAA module plugs directly into the mainboard adjacent to an expansion slot in the system chassis. When you remove the blanking plate from the system chassis, you can access the LINE and TEL RJ11 connectors on the metal edge of the Fax/Modem DAA module.

- Locate the J7 modem header on the mainboard.

- Plug the Fax/Modem DAA module into the J7 modem header.

- Remove the blanking plate adjacent to the Fax/Modem DAA module.

Modem DAA Module

Modem Header

Line & Tel

RJ11 Sockets

#### Optional Digital Audio Extension Bracket

This bracket has two RCA jacks for digital audio in and digital audio out, and an auxiliary jack for a stereo line-in device. It also provides a pair of optic fiber interface which enables the communication with MiniDisk or high-end audio systems.

SPDIF Out

Optic Out

Optic In

SPDIF In

Aux In

Digital Audio

Extension Bracket

J11 SPDIF Header

Aux In(L) 1

+12V 3

SPDIF/In 5

SPDIF/Out 7

2 Aux In(R)

6 GND

8 GND

- On the mainboard, locate the J11 SPDIF header for this bracket.

- Plug the cable from the bracket into J11.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Internal Digital Audio

If you have the optional internal digital audio cable, you can use it to connect the digital audio output connector of a CD-ROM or DVD drive to the pins 5-6 of J11.

#### Optional ATX Form Card

Infrared Port

USB Ports

ATX Form Card

PS/2 Mouse

Port

J1-ATX Header

1

This ATX Form card provides a mini-DIN port for infrared, one mini-DIN port for a PS/2 mouse. In addition it has two USB (Universal Serial Bus) ports.

- On the mainboard, locate the J1 ATX header for this bracket.

- Plug the cable from the bracket into the J1 ATX header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Optional Infrared Port

The mainboard has an infrared header IR1 so that you can install an optional serial infrared port.

1

IR1 Infrared Header

- On the mainboard locate the infrared port header IR1.

- Connect the ribbon cable from the port to the header IR1 and then secure the port to an appropriate place in your system chassis.

### Install Other Devices

Install and connect any other devices in the system following the steps below.

1

1

1

FDD1

IDE2

IDE1

#### Floppy Disk Drive

The mainboard ships with a floppy disk drive cable that can support one or two drives. Drives can be 3.5” or 5.25” wide, with capacities of 360K, 720K, 1.2MB, 1.44MB, or 2.88MB.

Install your drives and supply power from the system power unit. Use the cable provided to connect the drives to the floppy disk drive header FDD1.

#### IDE Devices

IDE devices include hard disk drives, high-density diskette drives, and CD-ROM/DVD drives.

The mainboard ships with an IDE cable that can support one or two IDE devices. If you connect two devices to a single cable, you must configure one of the drives as Master and one of the drives as Slave. The documentation of the IDE device will tell you how to configure for Master or Slave.

Install the device(s) and supply power from the system power unit. Use the cable provided to connect the device(s) to the Primary IDE channel connector IDE1 on the mainboard.

If you want to install more IDE devices, you can purchase a second IDE cable and connect one or two devices to the Secondary IDE channel connector IDE on the mainboard. If you have two devices on the cable, one must be Master and one must be Slave.

#### Internal Analog Sound Connections

If you have installed a CD-ROM drive or a DVD drive, you can connect the sound output of the drive to the built-in sound system.

On the mainboard, locate the two 4-pin connectors for J4 and J5. There are two kinds of connector because different brands of CD-ROM drive have different kinds of cable connectors on their audio output cable. Connect the cable to the appropriate connector.

J4

1

1

J5

### Expansion Slots

This mainboard has two PCI 32-bit expansion slots and one 8/16-bit ISA slot.

PCI1

ISA1

PCI2

Use the PCI slots to install 32-bit PCI expansion cards. Use the ISA slot to install legacy 8/16-bit expansion cards.

Note: The PCI2 slot can only be used if you disable the onboard LAN network using jumper JP6.

#### Installing an Expansion Card

- Locate the PCI or ISA slot on the mainboard.

- Remove the blanking plate from the appropriate expansion slot on the system chassis.

- Install the edge connector of the expansion card into the slot and press it quite firmly down so that it is seated correctly.

- Secure the bracket of the expansion card into the expansion slot in the system chassis using the screw that held the blanking plate in place.

#### Wake On LAN Connector

If you have installed a network adapter card, connect the adapter to the wake on LAN connector J8. You can then use the setup utility to program your computer to resume from a power saving mode whenever there is traffic through the network.

J8

1