February 1993

FTP PC/TCP Installation

for

D-Link DE-650 PCMCIA Ethernet Adapter

=========================================

The following files will be used in this installation:

Files	      Contents

-----------   ---------------------------------

DE650PD.SYS   Packet Driver Program

DE650PD.COM   Packet Driver Program

The D-Link packet driver programs above let you run FTP's PC/TCP

with the D-Link DE-650 series Ethernet Card. Two methods can be used:

Note:

Before run packet driver, run PCMCIA initial program on A:\OPEN_ADP

directory.

Please see as following:

*** PC installed Socket service, Card service, and Client driver ***

Skip this step

OR

*** PC installed Socket service or Databook Cardtalk ***

AE.EXE [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

OR

*** PC's PCMCIA controller is Intel 82365SL ***

PCIC.EXE [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

OR

*** PC is Toshiba T3300SL ***

ICCNT.EXE [/iop:P0] [/mem:P1] [/irq:P2]

*** For Phoenix PCM ***

DEVICE= PCMSS.EXE

DEVICE= PCMCCU.EXE

DEVICE= PCMDLINK.COM (irq:5, mem:d600, io base:320, only PCMCCU v1.3 need)

Please refer to \OPEN_ADP\README.DOC or \install.doc for detail information.

*** Parameters for OPEN-ADP utility ***

PCIC.EXE

PCIC.EXE is used to enable Intel 82365SL PC Card Interface Controller

(PCIC) chipset.

ICCNT.EXE

ICCNET.EXE is used to enable ICCNT (I/O Card Controller) chipset in

TOSHIBA T3300SL Model Notebook.

AE.EXE

AE.EXE is used to enable PCMCIA Ethernet Card on which Socket Service has

been installed.

PCMDLINK.COM

PCMDLINK.COM is PCM client driver and Phoenix PCMCCU v1.3 need.

device=ae.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

or

device=pcic.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3] [/grp:P4]

or

device=iccnt.exe [/iop:P0] [/mem:P1] [/irq:P2]

device=pcmdlink.com (resources is fixed on irq:5, mem:d600, io base:320

Only PCMCCU v1.3 need PCMDLINK.EXE

Parameter description:

/iop   :I/O base

/mem   :Attribute memory base

/irq   :IRQ value

/slt   :Slot type

/grp   :Group value

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable window memory base.

The default value is 0xd000.

P2 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default number is 1.

P4 :The selectable grp value.

The default value is 0.

Group 0 means Index IO is 3E0h, Data IO is 3E1h, Base is 00h

Group 1 means Index IO is 3E0h, Data IO is 3E1h, Base is 80h

Group 2 means Index IO is 3E2h, Data IO is 3E3h, Base is 00h

Group 3 means Index IO is 3E2h, Data IO is 3E3h, Base is 80h

METHOD 1:

If you use the DE650PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE650PD.SYS <-d> <-w> <sw_int> <irq> <io_addr> <m_base>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the DE650PD.COM program, follow the procedure below:

1. Execute the DE650PD.COM as follows:

>DE650PD <-d> <-w> <sw_int> <irq> <io_addr> <m_base>

2. Execute the ETHDRV.EXE device driver program (provided by FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<-d>		Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

<-w>		the -w parameter is for use with the Windows environment only.

<sw_int>	is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<irq>		is the hardware interrupt (IRQ) line set on your network

interface card. The default is 5 if this parameter is not

specified.

<io_addr>	is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter

is not specified.

<m_base>	is attribute memory base. The default value is 0xd000.