---------------------------------------------------------------------------

WINDOWS DRIVERS FOR THE PROAUDIO SPECTRUM PLUS VER 1.3

---------------------------------------------------------------------------

NOTE: Please read the WHATSUP.DOC for more information of the individual

files.

Because this is a update disk you will have to go through a few steps to

make sure you get a seccessfull install.

1. Remove all older drivers. This is done by running Windows and go in

the group called Main then run the Control Panual. Next double click on

the Drivers icon.

2. Remove all the drivers that relate to the ProAudio Spectrum and Media

Vision. Also remove [MCI] Mixer Control. There will be a total of 4 to

remove.

3. Return to the Program Manager. Now go in the Multimedia Tools group.

4. Select one of the icons (click once) and then hit the DEL key on your

keyboard. Then Enter. This will delete a icon. Continue for the next 3

icons. You do this so you wont end up with duplicate icons.

5. Restart Windows.

6. Select FILE then RUN and enter : A:\WINDOWS\INSTALL

7. Follow the install instructions that appear on the screen.

----------------------------------------------------------------------

PREVIOUS INFORMATION CALLED : README.TXT

----------------------------------------------------------------------

WINDOWS DRIVER UPDATE

May 6, 1992

This is the latest release of improved Windows drivers for the Pro

AudioSpectrum family of audio cards from Media Vision.

Please note:

1) There remain several outstanding bugs/features.

2) Additional testing, bug-fixing, and enhancements will continue to

be made.

We recommend that you make a backup of your existing drivers before

using the ones provided with this release.  If you discover any

further bugs, please notify us via CompuServe ID 75300,2772.

FIXED IN THIS RELEASE:

1) Support for hi-order IRQ's (above 7)

2) Using MCI player to play short .WAV files no longer skips

3) MIDI input no longer becomes disabled at random times (or at all)

4) WaveOutReset fixed, avoiding GP faults.

5) MediaPlayer 'thumb' not correct on playback of some WAV formats

6) Pro AudioSpectrum Plus owners may discover other fixes not

metioned above

TROUBLE SHOOTING:

1) DMA channels 0,5,6,7 don't work

WORKAROUND: Use DMA channels 1,2,3

PROBLEM: Your computer should be able to support 16 bit DMAs with

no problems. However we have found (even here in tech.

support) that some computers have a problem with 16 bits

things like the DMA controller has its own clock speed

and needs to be configured (consult your cpu manual). We

have even had a bad system where opon swapping in a new

system board fixed the problem. The software (ver. 1.2

or later, check your Media Vision Windows drivers disk

label) has been fixed to fully support the 16 bit DMAs

and IRQs.

2) Audio dropouts when running Microsoft Works, Kings Quest V,

Grandma and Me, and BattleChess.

WORKAROUND:  comment out VPASD.386 in system.ini

3) Recording causes system crash when using both VPASD.386 and any

one of QEMM, 386Max, EMM386.EXE

WORKAROUND 1:  comment out VPASD.386 in system.ini

WORKAROUND 2:  run Windows in standard mode

WORKAROUND 3:  remove memory manager

4) Original Pro AudioSpectrum induces low volume hi-frequency

system noise into 11 and 22 KHz recordings.

WORKAROUND 1:  record at 44KHz and convert down

5) New drivers don't work under Windows 3.0 with MME.

WORKAROUND:  migrate to Win 3.1; retain any unsupplied programs.

PAS16: Call cust. sev. and get DISK 6. There is a speial version

of mvproaud.drv you will need.

6) Pocket Recorder (not supplied in driver release) level

indicators don't support 16 bit audio.

WILL BE FIXED IN 1.4 WIN DRVS.

7) Pocket Recorder fails in 32K colors mode.

WORKAROUND: Load the sound drivers 1st then the graphic drivers.

8) Constant static.  Some motherboards have an unreliable crystal.

The Pro Plus and Pro 16 hardware

FIXED IN VER 1.3 OF WIN. DRVS.

9) If share.exe is loaded in autoexec.bat, remove it before

installing Media Vision DOS software

FIXED IN VER 1.2 DOS INSTALL.

10) The new OPL-3 chip is not being restored to Adlib compatability

mode on Windows exit

FIXED IN VER 1.2 OF WINDOWS DRIVERS.

11) DOS IRQ and DMA settings are not restored on Windows exit

FIXED IN VER 1.2 OF WIN DRVS.

FEATURES IN DEVELOPMENT

1) Support sub-sampled 16-bit playback on original Pro

AudioSpectrum

2) MCI MIXER language to support STATUS command

3) Pro Mixer wave meter give more accurate display

4) Relocatibility of port addresses

5) Multiple card support

6) SCSI prefetch cache (CD-ROM)

GENERAL NOTES

1) Central Point's BACKUP and PCTOOLS 7.1 are reported to be

installing their own VDMAD.386 over the Windows 3.1.

Central Point's VDMAD.386 has the wrong version number and will

prevent mvproaud.drv from loading.

2) After changing one or more MIDI drivers, check the control

panel's MIDImapper applet.  Channels 1 thru 10 should be patched

to the MIDI interface and channels 13 thru 16 should be patched

to the on-board synthesizer (MVFM.DRV or OPL3.DRV)

3) Pro Mixer has two extra sliders on the Pro-16 that are not

documented in the manual.  The BLSTR channel is the volume

selector for the Snd Blaster compatible DAC output.  The REC IN

slider controls the recording montor level of the sum of the

signals mixed for recording. RECORD is the sum of the signals

mixed for input to the mixer/DAC.

4) Pocket Recorder has greyed-out compression controls.  They will

be enabled when compression becomes part of the Multimedia

Windows architecture.

5) Much of the existing DOS software does not yet support hi-order

IRQ's or 16 Bit DMA's.

6) VPASD.386 is a virtual Pro AudioSpectrum driver.  Its main

purpose is to provide a 32K DMA buffer for digital audio

playback at high data rates.  Windows cannot otherwise supply

more than a 4K buffer which is not sufficient.  KNOWN BUGS 2 and

3 are related to using the VPASD driver.  You may remove VPASD

to avoid those bugs but may find that high data rate recording

and playback are not possible.

7) VPASD.386 can be used on the original Pro AudioSpectrum to

achieve Sound Blaster compatiblity.  On the Pro AudioSpectrum

Plus, VPASD.386 prevents simultaneous use of the Pro

AudioSpectrum DAC and the Sound Blaster DAC.  The Pro

AudioSpectrum 16 can run both DACs simultaneously and VPASD.386

serves to give each DOS box its own virtual Pro AudioSpectrum.

8) Tech Support is often difficult to reach.  The best way to get

through is to send a FAX with a detailed question.  Also provide

any relevent information (ie. system type, things you've already

checked, etc.  Also, if you should discover a problem but then

figure a solution, please let us know anyway.  Often your

success will be a helpful tip for others.