cwspeed.doc                                            Mar 12, 1994

CWSPEED - CW SPEED MEASURING PROGRAM DOCUMENTATION

CWSPEED.EXE was written to establish a computerized and definitive

method for calculating CW speed by two different methods.  (5 letters

per word, and 50 bits per word)

The program was specially written for use at the Palm Beach County

Hamfest High Speed International Morse Code Contest, held every

October.

The program is written in Power Basic (tm) which is an updated version

of a fast compiler type BASIC, formerly known as Turbo Basic.

The author's computer is an IBM Clone, running DOS 6.0, at 12 MHZ.,

using 80286 architecture without a math coprocessor.

I will be happy to furnish the source code to anyone who sends me a

SASE large enough to hold 6 computer output pages of text.

My address is:

Ted Herrman,  AE8G

2827 Finch Drive

Holiday, Fla. 34690

NOTES ON CALCULATIONS:

1.  One may calculate CW speed by counting the number of characters in

a continuous block of "copy".  Letters have a weight factor of 1,

numbers and punctuation have a weight factor of 2 since they tend

to take longer to send.  The total number of words is defined to be

the above sum counted in 60 seconds, divided by 5.

While this method is easy, it is imprecise.  Any given piece of

text may contain more or less numbers or punctuation, or an uneven

distribution of words with easy or difficult letters.

Nevertheless, this figure is calculated, displayed and labelled as:

"Apparent Speed".

2.  A second, more precise way to calculate CW speed is to count every

dot and dash and every piece of time between them and between

words.  In this program, the methodology assumes the classical

structure for CW.  That is, a strict definition for how the

characters are built, as follows:

The length of time required to send one dot is called a bit.

The length of time required to send one dash is 3.0 bits.

The length of time between adjacent parts of a character is devoid

of sound and has the exact length of one bit.

The length of time between adjacent letters is 3.0 bits.

The length of time between adjacent words (a space) is 4.0 bits.

With this architecture, a word is defined to be 50.0 bits.

(It so happens that the word "PARIS" followed by a 4 bit space

contains exactly 50 bits. )

P .--.   11 bits + 3 = 14    cum 14

A .-      5      + 3 =  8        22

R .-.     7      + 3 = 10        32

I ..      3      + 3 =  6        38

S ...     5      + 3 =  8        46

space     0      + 4 =  4        50

Note that from the instant the end of the last dot of the "s" is sent

to the instant the first dot of the following "p"  is sent, 7 bits of

soundless time exist.

The program calculates speed on the 50 bit per word principle by

counting every bit in a body of text copied by a contestant, dividing

by the amount of time required to take down the copy, and then

dividing by 50 and declaring this to be:

" Actual Speed".

The program displays both figures ( 5 char/word, and 50 bits/word) on

the screen.

If you use a computer to send the text, you will find that its

precision is very great.  Under those conditions, if you know the real

rate the computer is sending versus the speed you tell it to send, you

will find that the result of the calculation method above will agree

with the real rate that you have measured.

To measure the real rate your computer sends copy at, create or use the

PARIS.ASC file also appearing on this disk.  The file consists of the

word PARIS, followed by a single blank space, repeated many times.

Using a stop watch, count the number of times the word is sent in

exactly one minute.  Set a very high speed rate from your computer or

keyboard to get as much precision from this experiment as possible.

Estimate right down to the part of a word if you reach 60 seconds in

the middle of the word.  Divide the actual counted number of PARIS's by

the speed you told the computer to send at, to calculate a ratio.  You

will find that this ratio remains constant even at other speeds.

To use the program CWSPEED, a contest official keys in verbatim copy

provided by a contestant.  Once entered, the computer checks every

character to insure that only acceptable characters were taken down by

a contestant.  The ARRL Rules are well known and inflexible for the

conditions under which copy is to be taken down.  One part of them

describes exactly what characters are allowed or disallowed.  The rules

also spell out that a contest must provide at least 1 minute (60.0)

seconds of continuous copy.  Harry Lewis, W7JKJ gave me a copy of the

ARRL rules.  These rules must be followed to the letter in case a new

world CW speed record should be attempted.

In a contest, the officials announce the speed to be used to send

characters.  In the West Palm Beach Contest, a computer program is

used to send the copy. The computer is told the speed to send at by

touching the ALT + W key combination and then entering the desired

number of words per minute.  The program is called CW and was written

by Dave Freese, W1HKJ.  One can obtain it from Shareware sources.

The program refers to the above ALT W speed.  Based on this number, the

program also calculates exactly how much time the contestant required

to take down his or her copy.  The computer program checks to see if

the contestant actually provided a full 60 seconds of continuous copy

as required by contest rules.

WALK THROUGH OF SCREENS AND EXPLANATION

Display:

***********************************************************************

*          INTERNATIONAL MORSE CODE SPEED MEASURING PROGRAM           *

*                            Written by                               *

*                         Ted Herrman, AE8G                           *

*                          2827 Finch Drive                           *

*                       Holiday, Florida 34690                        *

*                          Tel 813-937-5931                           *

*                      Version 2.0  Mar 9, 1994                       *

*                                                                     *

*   Push CTRL + BREAK key combination for immed exit at any time.     *

*                                                                     *

*                Touch any key to proceed:                            *

***********************************************************************

Explanation:  The authors ego line

Display:

***********************************************************************

*  Enter bits per word. (PARIS = 50) is standard. <CR> = default = 50 *

*                            Written by                               *

*  <CR> means:  Touch CARRIAGE RETURN or ENTER key.                   *

***********************************************************************

Explanation:

Unless you use a military standard or some other alphabet, 50 bits is

the standard number of bits in use throughout the United States.

Touching the ENTER key will cause the program to use 50 as a default.

Display:

***********************************************************************

*  Do you wish to produce calculations or exit the program?           *

*  Enter either <CR> for calculations or 'EXIT".                      *

***********************************************************************

Explanation:

This is an opportunity to gracefully exit the program.  There are other

ways to exit the program, and no harm will result from the use of any

of them.

Display:

***********************************************************************

*  Enter computer CW program average speed setting - (Alt W speed.)   *

*  This must be an integer from 5 thru 100.                           *

***********************************************************************

Explanation:

Enter the number of words per minute that the transmission was sent at.

If you use program CW, then enter the number you keyed in after first

touching ALT + the W key.

Display:

***********************************************************************

*  Enter  ACTUAL  /  REQUESTED  speed ratio.  Use a decimal point.    *

*  No. of times PARIS in 60 seconds  /  Requested computer speed.     *

*  For example:  Actual speed   58.0 wpm                              *

*                               ----          then ratio = .96667     *

*             Requested speed    60                                   *

*                                                                     *

*  You only have to enter this once per program execution.            *

***********************************************************************

Explanation:

If you use a computer or keyboard, determine the actual number of times

it sends the word PARIS, (followed by a single blank), with as much

precision as you can.  Divide this number by the speed you told it to

send at, to make a ratio.  You will find that in the case of a

computer, this RATIO will hold constant later on, even if you change

the requested speed.

Because of the constancy of the rate, you need only enter this ratio

once.  Program logic will avoid subsequent requests for you to enter

the number again.  If for some strange reason, the ratio does not hold

constant, you will have to start the program from scratch so the logic

is executed fresh, and asks you for the ratio.  (Programmers would

probably want to modify the code in this event, and I will make it

available to you, if you send a SASE.)

Suppose for example, you ask your computer to send the PARIS file at a

rate of 95 wpm.  By actual count, you count 98 occurrences of the word

PARIS and PA of the 99th word.  PA contains 22 bits of the total 50

bits and is .44 of a PARIS.  Then the ratio is 98.44 / 95

Enter 1.0362 at the ratio prompt.  5 significant digits are more than

enough to preserve accuracy.

Display:

***********************************************************************

*  Enter the contestant's lines of copy, including multiple spaces.   *

*  Take as many separate lines as necessary.                          *

*  Enter as many as 80 characters per line.                           *

*  For multiple line input, change lines in the MIDDLE of a word.     *

*  Signify end of input with 1 asterisk on a separate line.           *

***********************************************************************

Explanation:

Re key in exactly what the contestant copied.  Note that the contestant

apparently made an error in the last word (antenna).  See below.  You

must scan the input for errors and stop keying in at that point.  Look

for long strings of good consecutive copy.  Make your input IDENTICAL

in every regard to what the contestant copied with the exception that

you will make your carriage returns in the middle of a contestants

word.  This will insure that no extra blanks appear where they should

not.  This would penalize the contestant unfairly.  Also note in the

example below that the contestant put two blank spaces after the end of

the first sentence and one blank space after the end of the second.

FOR EXAMPLE: The following is a picture of what a contestant wrote:

"Although the extended double Zepp antenna has been in just about

every antenna handbook since the year one, hams seldom use it.  Its

overall length is 1.28 wavelengths, and it radiates bidirectionally

broadside. Fed with open wire line and a balanced antenna tuner, it

also makes a fine multi band anten"

HERE IS WHAT YOU KEY IN: NOTE THE LONE ASTERISK AS THE LAST LINE.

Although the extended double Zepp antenna has been in just ab

out every antenna handbook since the year one, hams seldom us

e it.  Its overall length is 1.28 wavelengths, and it radiate

s bidirectionally broadside. Fed with open wire line and a bal

anced antenna tuner, it also makes a fine multi band anten

*

The computer will ask you to verify that what you have keyed in, is an

exact match for the contestants copy.  Reply "YES" or "y", otherwise

you will have to re-enter the contestants entire copy again.

PROGRAM OUTPUT

1.  The computer checks each and every character to verify that no

illegal characters were sent and/or copied.

Legal characters are:  alphabet, digits, comma, period, question

mark and fraction bar (slash).  Nothing else is legal, according to

ARRL Rules.

If no illegal characters are found, the screen will show:

"All input is legal."

If illegal characters are found, the entire text is put on the

screen and the illegal characters are identified.

2.  The program displays the total number of bits counted, the total

words counted, the total characters counted, and the amount of time

the contestant needed to copy the text entered.  From these

statistics, one may manually verify the displayed, calculated

speeds that follow on the same screen.

I wish to acknowledge the assistance given to me by Harry Lewis, W7JKJ.

Harry is a living legend and a goldmine of advice about CW contests and

procedures and CW speed measuring algorithms.

Any inaccuracies or omissions in the program are my responsibility

alone.  Please write with your comments and criticisms and etc.

73 and di di di dah di dah.

Ted  Herrman,  AE8G.

#####################################################################################################################################################################################################################################################################################################################################################################################