User's Guide

EScan.exm  Ver 0.4

Toshiki Sasabe

<toshiki@j.email.ne.jp>

[1] Introduction

Escan.exm is a scanner utility for the GT-6000/ES-300C

scanner from Epson.  It controls the scanner and saves the

scanned image in BMP, ICN or PCX format on the HP 100/200LX.

[2] Files

This package contains the following files:

ESCAN.EXM   Program

ESCAN.DOC   User's Guide (in English)

ESCAN.JPN   User's Guide (in Japanese)

ESCAN.ICN   Icon file

ESCAN.C     Source code

ESCAN.H     Source code

BMPPCX.H    Source code

[3] Connection

1) The scanner should connect to the 200LX with a serial

cable.  Use the following combination.

o HP connection cable for PC (Part number F1015A)

o HP adapter for printer connection 5181-6640 (9-to-25

adapter, with a printer mark, included in HP connector

kit F1023A)

2) Adjust the DIP switch SW1 on the scanner (switch for

connection settings). For example:

SW 1-Bit 1  OFF  \

SW 1-Bit 2  OFF   > 19200 bps

SW 1-Bit 3  OFF  /

SW 1-Bit 4  OFF  Stop bits =1

SW 1-Bit 5  ON   Even parity

SW 1-Bit 6  ON   Enable parity check

Other condition can be used.  Make sure it is the same as

the settings on the 200LX (See below).

[4] Software installation

1) After a scan cycle, ESCAN displays the scanned image by

use of an external program, LxPic.  You need to install

LxPic for this function to work.  (You can switch off this

display function, by [Menu] -> Option -> Other)

2) Copy the file ESCAN.EXM to your preferred directory.  Add

it to MoreEXM or to AppManager.

3) Connect the scanner to the 200LX, power up the scanner and

run the program.

4) A "Communication Settings" dialogbox will appear. Enter

the settings, for example:

Baud       19200

Interface  Com1

Parity     Even

Data Bits  8

Stop Bits  1

5) Press [F10](OK) to close the dialogbox. ESCAN will send

the RESET command to the scanner and start communicating

with it.

6) After 10 to 20 seconds, ESCAN will display "Status: Ready"

at the first line of the display.

Note 1: If the connection fails here, reconfirm the

setting of the scanner and the cable connection,

select [Menu] -> Option -> Communication and

repeat from step 3).

Note 2: When you start the program next time, the same

communication settings will be used. Always make

sure that the scanner is powered on and connected

to the 200LX before running the program.

7) Select [Menu] -> Option -> Other, enter the directory to

hold the image data and press [F10](OK).

Note: MoreEXM is a freeware utility to enhance the System

Manager, created by Hiroyuki Sekiya.  LxPic is a

freeware image viewer created by Stefan Peichl.

[5] Usage

5.1 Basics

1) Follow the steps:

- Press [F7](Color) and enter the color settings.

- Press [F8](Area) and enter the scan range.

- Press [F9](Start) to start the scan.

2) Repeat 1) if you need, then

- Press [F10](Quit) to exit from the program.

5.2 Detail of Color Settings ([F7])

Color Mode

+-Monochrome

+-Color (1 scan)  line interleave

+-Color (3 scan)  plane interleave

Drop out            Drop-out color

+-None

+-Red

+-Green

+-Blue

Image Depth         bits per pixel per color. 1 to 8.

Dither              Works with the Image Depth of 1 to 2.

+-None

+-Normal

+-Soft

+-Halftone

Brightness          -3 to +3

Gamma Correction

+-CRT-A           No Correction

+-CRT-B

+-Printer-A       High resolution printer

+-Printer-B       Low resolution printer

+-Printer-C       text and images mixed

Color Correction

+-None

+-Impact Printer

+-Thermal Printer

+-Inkjet Printer

+-CRT Display

5.3 Details of Area Settings ([F8])

Save as    Filename.  Default is PIC001, PIC002, PIC003...

A warning is displayed, if a file with the same

name exists.

Format     BMP, PCX or ICN. (ICN and PCX works for a

monochrome image with the depth of 1)

Resolution DPI (dots per inch)

Zoom       Per cent unit. From 50 % to 200 %.  X=100%,

Y=84% for the 200LX display.

Scan from  the position of the upper left corner of the

image

Scan Area  Width and height of the image to be scanned.

Lock X=Y   Mark here to synchronize the X and Y values

for Resolution and Zoom settings.

5.4 Menu

+- Action   +- Start          Same as [F9](Start)

|           +- Scan Color     Same as [F7](Color)

|           +- Scan Area      Same as [F8](Area)

|           +- Reset          ESCAN will send the RESET

|           |                 command to the scanner and

|           |                 start communicating with it.

|           |                 Same as [F3](Reset).

|           +- Exit           Same as [F10](Quit)

|

+- Option   +- Communication

|           +- Other          Directory setting and LxPic

|                             switch.

|

+- Quit                       Same as [F10](Quit)

|

+- Help     -- About          About this program

[6] Examples of image settings

(1) If you want to scan a photograph to view it on the 200LX

(monochrome 1-bit):

Color Mode        Monochrome

Image Depth       1

Dither            Normal

Brightness        0 (adjust for the image too dark or

too bright)

Gamma Correction  CRT-B

Format            PCX or ICN

Resolution        X=Y=144 DPI

Zoom              X=100% Y=84%

This setting create an image slightly larger than the real

size.

(2) If you want to scan a text, graph or line-arts to view it

on the 200LX(monochrome 1-bit):

Color Mode        Monochrome

Image Depth       1

Dither            None

Brightness        0 (adjust for the image too dark or

too bright)

Gamma Correction  CRT-A

Format            PCX or ICN

Resolution        X=Y=144 DPI

Zoom              X=100% Y=84%

(3) If you want to scan a color photograph in full color (RGB 24 bit)

to view it later on other PC.

Color Mode        Color 1-scan

Image Depth       8

Dither            None

Brightness        0 (adjust for the image too dark or

too bright)

Gamma Correction  CRT-B

Format            BMP

Resolution        X=Y=120 DPI

Zoom              X=Y=100%

When used on a normal size picture (130 mm x 90 mm),

this resolution gives you 610x425 pixel image, slightly

smaller than VGA screen (640x480).

[7] Limitations and other remarks

BMP format creates the images upside-down.  Place the scanned

object upside-down to make a correct image.

Settings are stored in a file C:\_DAT\ESCAN.ENV.  You can

reset the program to the default status by deleting this file,

Consult the user's guide of the scanner, if you are not sure

about the functions of each settings.

Color images are stored always in 24-bit BMP format.  It

tends to be very large (3 bytes for one pixel).

[8] Acknowledgement

Encoding routine for PCX format was copied from CAPSYS.C,

written by Hiroyuki Sekiya. I would like to thank Mr.Sekiya

to permit me to use his code.

[9] Copyright, disclaimer and distribution policy

This software is a copyrighted freeware.  Its copyright is

owned by its author Toshiki Sasabe (toshiki@j.email.ne.jp).

The author is not responsible for any damages, which may be

caused by the use of, or inability to use, this software.

This software can be distributed freely under the following

conditions.

o The distribution should be for non-commercial purposes.

o The entire archive should be distributed intact.

Prior permission from the author is needed for distributions,

which do not meet these conditions.

[10] Revision History

Ver 0.1 o Internal test version

Ver 0.2 o First release (NIFTY SERVE)

Ver 0.3 o First release on Internet

o Interrupt with ESC key now shows an messagebox

correctly.

o [Menu]->Action->Start now works.

o Color 3-scan now works

o "Replace file?" dialog added.

o A lot of other small improvements

Ver 0.4 o Selection of "CRT Display" in Color Correction

listbox now works.

o Removed a bug that a datablock longer than 512

bytes hangs the program.