Documentation for JRHOTKEY

Multi-Purpose  PC-Compatibility  Driver  for the  IBM  PC-JR,

PC  Convertible,  PC JX;  also has uses  on  EGA,  VGA.  Sets

Background Colors for Graphics-Mode Programs.

by Robert Sheaffer,  SHEAFFER at SCRVM1

IBM Internal Use Only

Release History:

RELEASE 1:       May 13,  1988:  "Hot Key" with two  graphics and

one text mode. Default color is blue.

RELEASE 2:       June 15,  1988:  Added command-line parameter to

choose default color palette.

Added  support  for the PC-JR  16-color  medium-

resolution  mode,  and  4-color  high-resolution

modes.  If run on a non-JR machine,  these modes

change  to the corresponding 16-color EGA medium

and  high-resolution modes.  The LEFT-SHIFT  key

functions as before;  adding the RIGHT SHIFT key

in  the same keystroke enters the special  PC-JR

or EGA enhanced color modes.

Added  CTL-BOTH  SHIFT-J for entering text  mode

from a 62-key PC-JR keyboard, which disables the

CTL-ALT-SHIFT-J combination.

RELEASE 3:       June 29,  1988: Added JRHRDKEY to package, which

prevents  "stealing"  of the  NMI  vector.  This

allows  QUICK-C and other NMI-grabbing  programs

to be used without any patches. Also, in certain

limited  circumstances,  JRHRDKEY will survive a

reboot,  allowing it to be used with some  self-

booting diskettes.

RELEASE 4:       Oct. 10,  1988.  JRHOTKEY  now checks to see the

|                type of system it is installed on; if it is a PC

|                JR or PC Convertible ("laptop"), it installs the

|                "NMI  Protector"  originally only  in  JRHRDKEY,

|                unless specifically instructed not to.  JRHRDKEY

|                is  now obsolete,  but is not removed  from  the

|                package.

FUNCTION:

JRHOTKEY fixes scrambled graphics screens on the PC-JR,  PC-

|    JX,  and on the EGA. It also prevents the redirection of the

|    NMI  handler  on  systems that utilize  the  NMI  for  their

|    keyboard interrupt.  It was derived in part from PCINT09  on

PCTOOLS, which fixes the compatibility problem caused by the

PC-JR's  failure to place the keystroke scan code in AL upon

entry to the INT 09 routine.  This fix allows many programs,

including  Microsoft's QUICK BASIC and QUICK-C,  to  operate

correctly on the IBM PC-JR.

JRHOTKEY retains the fix for the INT 09 function, and adds a

"hot  key" capability which,  when confronted by a scrambled

graphics-mode  screen  caused  by  the  programmer   writing

directly to the CGA's hardware, forces the PC-JR into a CGA-

compatible  graphics  mode.  If  the  display  is  scrambled

because  of  an inappropriate value written  to  the  System

Control Port,  whether in graphics or text mode, it will fix

that,  too.  It  also resets the diskette controller,  which

will forces an endlessly-spinning diskette to stop spinning.

This  program  is  also sometimes useful in  "rescuing"  CGA

programs  that refuse to run on the EGA,   or have miserably

bad contrast when running there, owing to the EGA's extended

color palette.

INVOCATION:

JRHOTKEY X

Where   "X"   is  an  optional   hexadecimal   digit   (0-F)

representing  the  initial  color  setting  of  the  default

palette.  This  allows  any one of the Jr's 16 colors to  be

selected as the initial background color for graphics modes.

This  value  may be 0-9,  or A,B,C,D,E,  or F.  If it  is  a

letter,  it  must be a capital letter.  (For information  on

which  numbers  represent  which  colors,   see  the  "COLOR

statement - text mode" in your BASIC manual).  If an invalid

character  is specified,  the background color will  be  set

randomly.  If  no parameter is specified,  the initial color

will default to 1 (blue).

|          JRHOTKEY H

|              -or-

|          JRHOTKEY H X

|

|    When the first (or only) parameter is the uppercase "H",  it

|    instructs  JRHOTKEY to keep "hands off the  NMI."  Normally,

|    NMI  Handler  Protection is installed whenever  the  machine

|    type is found to be a PC JR, PC Convertible, or PC JX. These

|    are  the  machines  that  use the  NMI  for  their  keyboard

|    interrupt,  and  if the NMI Handler is diverted (because the

|    programmer  thought  that the NMI would be invoked  only  by

|    unwanted  math  coprocessor  interrupts),  the  keyboard  is

|    disabled.   Users  of the PC JR,  PC JX,  and PCC  typically

|    should NOT try   to disable this funtion, and users of other

|    systems  need not  be concerned about it,  as this  function

|    will not be  installed on their machines. There are a few PC

|    JRs with  special modifications,  such as PCKBONJR,  or with

|    Math   Coprocessors,  that  may  need  to  disable  the  NMI

|    Protection to  utilize those features,  but these customized

|    systems  are  very few in number.  Even if you have  an  XT-

|    compatible   keyboard  attached  to a PC Jr via  the  Racore

|    adapter,  or  a  similar external keyboard adapter,  the NMI

|    Handler is still being  utilized,  so you should not disable

|    the NMI Protector.   JRHOTKEY notifies the user when the NMI

|    Protector  Function   is installed.  This  function  remains

|    active  as  long  as  the  system  is  running  and  is  not

|    rebooted.

"HOT KEY" USE:

KEY COMBINATION           ON PC JR / JX       ON OTHER MACHINES

ALT-J                   320 x 200                 SAME

MED-RES GRAPHICS

ALT-LEFT SHIFT-J         640 x 200                 SAME

2-COLOR GRAPHICS

CTL-ALT-LEFT SHIFT-J     80 X 25 CHARACTER         SAME

-or-               TEXT MODE

CTL-BOTH-SHIFT-J

ALT-J + RIGHT SHIFT      320 X 200 16-COLOR   320 X 200 16-COLOR

PC-JR GRAPHICS       EGA/VGA GRAPHICS

ALT-LEFT SHIFT J +       640 x 200 4-COLOR    640 x 200 16-COLOR

RIGHT SHIFT              PC-JR GRAPHICS       EGA/VGA GRAPHICS

DESCRIPTION:

This program constitutes,  to the best of my knowledge,  the

most complete existing "PC Compatibility driver" for the PC-

Jr.  It should be loaded by your AUTOEXEC.BAT file, and then

ignored  until  needed.  It  is  well-behaved,  and  has  no

observable  effect  unless  the "hot  key"  is  pressed.  In

essence,  it functions as a ROM BIOS enhancement for greater

video mode PC-compatibility.

This routine is provided on an as-is basis with no  warranty

(or  support)  expressed  or implied and is  classified  IBM

Internal  Use  Only.  This  is considered to  be  incidental

programming and is made available as such.  If any  problems

are  reported  to  me,  I  will deal with them  on  a  'time

available' basis.

This  program makes the PCJr keyboard interrupt routine (INT

09h) compatible with the PC keyboard interrupt routine.  The

difference between the two is that the PC routine takes  its

input  from  hardware port 60h while the PCJr routine  takes

its  input  from  the  AL  register  (supplied  by  the   PC

compatibility  routine  INT  48h).  The  PCJr  compatibility

routine  puts the converted key scan code into hardware port

60h just prior to calling the INT 09h routine.  This routine

loads  and keeps resident a small routine that ensures  that

the BIOS INT 09h routine takes its input from hardware  port

60h, thus creating strict PC compatibility.

The  PCINT09 routine was originally developed by Max  Maurer

(MAURER  at POKVMCR3) to allow QUICKBASIC 3.0 to run on  the

PCJr.  QB  3.0 intercepts INT 09h and the mis-match  between

the   standards   caused  random  errors  such   as   double

characters,  ignored characters, etc. Since this routine has

a null effect on the PCJr when inserted directly between the

PCJr BIOS routines INT 48h and 09h,  it can be installed any

time  and  just  left  there  without  affecting  any  other

programs running on the PCJr. If you are using JRHOTKEY, you

should  not  also  use PCINT09 at  the  same  time,  as  its

function is included within JRHOTKEY.  QUICK-C also requires

this same "fix" to run on the PC Jr.

In addition to the INT09 PC-compatibility function described

above,  JRHOTKEY performs three additional functions when  a

"hot key" is pressed:  A), it forces a graphics-mode program

which   writes  directly  to  the  CGA  hardware  into   the

appropriate  graphics  mode using BIOS  calls,  which  often

allows an otherwise-unreadable screen to be correctly viewed

on  the PC-JR;  B),  because in some cases direct I/O to the

PC's  floppy  diskette  controller leaves the PC  JR's  disk

spinning endlessly,  the JR's diskette controller is  reset;

and  C),  when in text mode,  it ensures that an appropriate

value  is  written to the Junior's System  Control  Port  B,

which is another major source of PC-incompatibility.

The  video reset function is performed upon detection of the

keystroke function ALT-J,  ALT-LEFT SHIFT-J, or CTL-ALT-LEFT

SHIFT-J.  (The RIGHT SHIFT key may be added when going  into

graphics  modes  to  get  the  corresponding  enhanced-color

mode.)  This  is  useful when you are faced with  a  display

which is scrambled,  blank,  or otherwise unreadable. (Video

incompatibilities  may  arise when in text  mode,  owing  to

inappropriate values being written to System Control Port B,

at   address  61  Hex.)  Scrambled  graphics  displays   are

generally  a  consequence of the programmer having  set  the

video mode by writing directly to the CGA's video controller

hardware, instead of using the BIOS functions (as if the few

microseconds  saved  ONCE  by this  action  were  worth  the

incompatabilities  it causes on the PC JR,  EGA,  and VGA!).

The keystroke ALT-J forces you into Video Mode 5 (320 x  200

4-Color Graphics). The keystroke ALT-LEFT SHIFT-J forces you

into  Video Mode 6 (640 x 200 2-color graphics).  These  are

the only two graphics modes available to programs written to

be  run on the CGA.  The display is NOT cleared,  so that in

the  great  majority  of cases the  scrambled  display  will

simply  jump  "into  focus" upon hitting one  of  these  two

keystrokes.

You  can  enter  the PC-JR's enhanced color  modes  (or  EGA

enhanced  modes) by "adding in" the RIGHT-SHIFT key  to  the

above.  Thus,  ALT-RIGHT SHIFT-J puts you into 320 x 200 16-

color  mode  (Video Mode 9),  and ALT-BOTH SHIFT-J puts  you

into 640 x 200 4-color mode (video mode 0A).  This  probably

allows  some software written in the 16-color mode for Tandy

machines which now fails on the PC-Jr to run on it. You must

have  at least a 32K video buffer to enter either  of  these

modes  ( "/C" in PCJRMEM for IBM sidecar users,  "/v=32" for

Microsoft  Sidecar users).  When one of  the  enhanced-color

modes   is   entered,   JRHOTKEY   looks  at   the   Machine

Identification byte in the ROM BIOS to determine the kind of

machine it is running on (see TELAPART FORUM on  IBMPC).  If

the  machine  type = PC JR (0FD hex),  it will enter  either

video mode 09 or 0A. If it is anything else, it assumes that

an EGA is present (otherwise,  why would you be selecting  a

non-CGA mode?), and adds 4 to the above video modes, putting

you into either 320 x 200 16-color mode (video mode 0D),  or

640 x 200 16-color mode (video mode 0E).  When run on a PS-2

with a VGA, JRHOTKEY functions the same as on the EGA.

The keystroke CTL-ALT-LEFT SHIFT-J forces you into text mode

if you are in graphics mode,  and clears the screen;  if you

are  already  in  text mode,  all it does  is  reset  System

Control Port B. Because the CTL-ALT sequence is decoded in a

special  way  by the PC JR's unique 62-key  small  keyboard,

CTL-LEFT SHIFT-J will not work when using that keyboard.  As

a  substitute,  the sequence CTL-BOTH SHIFT-J (without  ALT)

has been programmed to do the same thing. Either combination

may be used on the XT,  AT, or PS-2 keyboards, or on a PC-JR

to  which  an XT keyboard has been attached via  the  Racore

adapter.

|    When JRHOTKEY or JRHRDKEY are installed,  if the system type

|    is PC JR,  the "initial video state" in the BIOS Memory Area

|    is  changed  from  "40 Character  Color"  to  "80  Character

|    Color".  The problem is that some programs check to see if a

|    CGA  or equivalent is installed before they try to run,  and

|    unless  they find an initial mode of "80  Character  Color",

|    they refuse to run.

There are some programs for which JRHOTKEY will not succeed,

such as when the JR's video hardware is continually hit with

inappropriate I/O,  or when the program grabs control of INT

09,  thereby locking us out! However, in most cases it works

as  planned,  opening up a great many programs to be run  on

the PC JR which had previously been completely unusable. For

example,  two  public-domain astronomy programs I  obtained,

one  whose  display was invisible on the PC  Jr,  the  other

nearly so, can now be "salvaged" by the use of ALT-J, as can

many games,  and the graphics of the old, pre-Jr versions of

LOTUS 123.  Some other examples:  Prodesign II now works  on

|    the  PC-JR using the "hot key",  as does Microsoft  Windows,

and the game "Digger" can now be played on the PC-JX.  There

are  undoubtedly many more programs to be fixed by the  "hot

key" which have not yet been tried.

Because  of  the different color palettes of the CGA and  PC

JR,  some programs (especially certain ones written in Turbo

Pascal)  produce a graphics screen  which,  while  nominally

compatible  with the JR's display,  are of such low contrast

(for example, cyan or white on bright yellow!) that they are

impossible  to see clearly.  ALT-J or ALT-LEFT  SHIFT-J  (as

appropriate)  changes  the background  color,  bringing  the

screen  into  sharp  contrast!   You  can  also  change  the

background  of  graphics-mode programs for purely  aesthetic

reasons. For example, I have several programs written to run

against dull,  drab black backgrounds that I much prefer  to

see as blue!

You  can set the initial background color as a  command-line

parameter when invoking JRHOTKEY. Use a single hex digit, 0-

F.  If no color is specified, the background color is set to

blue.   If  you  want  to  view  other  background   colors,

repeatedly pressing one of the graphics hot-key combinations

allows  one to cycle through the JR's 16-color palette until

a desirable background color is reached.  The color  palette

remains  at the value you choose until you press a  graphics

"hot  key" combination several times in succession,  without

pressing any other key in between.

Each   time  the  "hot  key"  is   pressed,   the   diskette

controller  of  the PC JR is reset,  which will stop a  disk

which  is spinning aimlessly.  On the PC,  PC-AT,  and PS-2,

this  apparently has no effect,  so users who wish to  "pop"

scrambled  EGA  screens  into place will  not  find  this  a

hindrance.

There  are,  it turns out,  a few programs for which the key

combination  "ALT-J"  already  means  something.   Should  a

technically-proficient  user  wish to change the  "hot  key"

from "J" to some other key,  that key's "scan code" (NOT its

|    ASCII  value) is found at offset 2F9 hex into  the  program,

using DEBUG. At that location you will find 24 hex, which is

36  decimal,  which may be replaced with the "scan code" for

some other key.  If,  for example,  you wanted to change the

"hot  key" to be ALT-TAB,  ALT-SHIFT-TAB,  etc.,  you  would

patch  that location to be 0F hex,  the "scan code" for  the

TAB key. This is done as follows:

DEBUG JRHOTKEY.COM

|         - e 2F9

- (It types "24". You type "0F")

- w

- q

Other "scan codes" you may want to consider (all are hex): Z

= 2C;  Q = 10;  V = 2F.  The keyboard "scan codes" are given

in the Technical Reference Manuals for the PC/XT/JR.

The offset to be patched in JRHRDKEY is 3C0 hex.

CAUTION:

If accidentally installed twice,  JRHOTKEY will produce some

VERY strange symptoms!

If at all possible, try to avoid hitting the "hot key" while

the PC Jr diskette is actually transferring data. While this

is not invariably fatal - sometimes DOS can recover from the

unexpected  resetting  of the diskette controller  - it  can

occasionally  necessitate  a reboot.  Of  course,  when  the

diskette  is stuck in a state where it spins  endlessly,  no

data is actually being transferred, so go ahead and hit that

hot key!

Remember  not to try to set any of the enhanced-color PC  Jr

video modes unless your video buffer is at least 32K.

APPENDIX:

1.               JRHRDKEY - Obsolete Version of JRHOTKEY

The  module  JRHRDKEY.COM  represents a  special  "hardened"

version  of  JRHOTKEY.  The "hot key" function  in  each  is

identical.  However,  there  are two significant differences

with JRHRDKEY: First, it installs itself in low memory below

the  BIOS Memory Area,  at segment  20,  offset  0,  thereby

effectively  taking no memory from DOS  ;  Second,  JRHRDKEY

supports an optional command-line parameter "R",  which will

force  a warm reboot;  upon reboot,  JRHRDKEY will still  be

installed unless it is explicity overwritten or  re-vectored

by  the program (unfortunately,  most self-booting diskettes

do  seem to re-initialize the  interrupt  vectors,  although

some  do  not.  Testing suggests this feature will  probably

prove to have limited utility.  There will probably be  some

self-booting  programs,  however,  which it will "fix".) The

"R"  parameter,  if  used,  must  be  the  first  (or  only)

parameter,  and you must install JRHRDKEY using DOS 2.1 (the

reason for this is that DOS 3.X "hooks" INT 13 away from the

BIOS,  leaving  upon  reboot a pointer  to  nowhere!).  When

prompted to insert a self-booting diskette,  you may  insert

any such diskette, including DOS 3.X. (Example: JRHRDKEY R E

soft-boots  a  diskette,  leaving a "hot key" with a  yellow

background.)

When  the warm boot function is desired,  you must not  have

any "pop up" or TSR programs installed, and you must not use

any PCJR memory drivers such as PCJRMEM or MEMORY.SYS  (only

128K must be enabled,  no drivers or TSRs of any kind).  You

must  boot from a "plain vanilla" DOS 2.1.  The installation

of  a tiny TSR program in low memory is most  certainly  NOT

supported by DOS, yet seems to work just fine with the great

majority of programs, nonetheless. (JRHOTKEY, by comparison,

|    operates  in fully supported ways.) Because it is  installed

|    in  low memory,  JRHRDKEY is NOT compatible with the use  of

|    the PC JR  BASIC Cartridge;  interpreted BASIC programs will

|    fail to work if JRHRDKEY is installed.  Also,  JRHRDKEY does

|    not support the "H" parameter, telling it to keep "hands off

|    the NMI".

|    JRHRDKEY  is now obsolete,  since all its functions  (except

|    "warm reboot") are now included in JRHOTKEY.