JAGUAR II 386 Motherboard

Rev. 2.0

Supplementary

======================

1.	The  following options are cancelled:

|  | Reference |
| --- | --- |
| Wait for <F1> if any Error | Page A-11 |
| Page mode | Page A-11 |
| Cache with wait state | Page A-11 |
| 8 bit AT cycle wait state | Page A-11 |
| 16 bit AT cycle wait state | Page A-11 |
| DRAM Wait state | Page A-11 |

2.	The following options are added:

|  | Reference |
| --- | --- |
| System ROM shadow F000,64K | Page A-11 |
| Video ROM shadow C000,32K | Page A-11 |
| Adaptor ROM shadow C800,32K | Page A-11 |
| Adaptor ROM shadow D000,32K | Page A-11 |
| Adaptor ROM shadow D800,32K | Page A-11 |

3.	In the Jaguar II Rev 2.0, there are some features modified in the system, So the BIOS is added a ADVANCED CHIPSET SETUP option for selecting each feature. The following shown the main screen for ADVANCED CHIPSET SETUP:

BIOS SETUP PROGRAM - ADVANCED CHIPSET SETUP

<C> 1990 American Megatrends Inc., All Rights Reserved

Cache Select					:	64KB

C0000 -> DFFFF			:	disabled

Size of Non-Cache Area	:	256KB

Non-Cache Start Address	:	disabled

Allocation Non-Cached Area	:	sys RAM

Bus Clock Speed			:	sclk/8

DMA Clock Select			:	7.16MH

AT Wait State (see help)	:	Best

I/O Recover Time (see help)	:	Best

╔═════════════════════════════════════════════════════╗

║                                                     ║

║                                                     ║

╠══════════════════════════╦══════════════════════════╣

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

╚══════════════════════════╩══════════════════════════╝

a.	C0000->DFFFF Cachable

The C0000->DFFFF cacheable option is defined for the Video adaptor and some add-on card which locate in C0000->DFFFF. This option lets you set the address range to be non-cachable easily without concerning about the base address and size. The default is "disabled". If this option is enabled, the performance of the video BIOS is better,but it is likely that the BIOS may be overwritten by other software.

b.	Size of Non-Cachable Area and Non-Cachable Start Address

Non-cachable start address and size can be defined. The size of non-cachable area  may be 256K, 512K, 1M and 2M. The start address is where the non-cacheable size will start in the main memory. Press 'PgDn' and 'PgUp' keys to change the address value to an appropriate value.

c.	Allocation Non-Cached Area

SYS DRAM :

The non-cachable area applies to the on-board system.

AT Bus :

The non-cachable area applies to the AT bus.  On-board system DRAM in the area is disable.

4.		The following option is improved:

Memory Configuration

256KB,1M and 4M SIMM are support. The following memory configuration are added:

| Bank 0 | Bank 1 | Bank 2 | Bank 3 | Total |
| --- | --- | --- | --- | --- |
| 256K | --- | --- | --- | 1M |
| 256K | 256K | --- | --- | 2M |
| 1M | --- | --- | --- | 4M |
| 256K | 1M | --- | --- | 5M |
| 256K | 256K | 1M | --- | 6M |
| 1M | 1M | --- | --- | 8M |
| 256K | 1M | 1M | --- | 9M |
| 256K | 256K | 1M | 1M | 10M |
| 1M | 1M | 1M | --- | 12M |
| 1M | 1M | 1M | 1M | 16M |
| 4M | --- | --- | --- | 16M |
| 4M | 4M | --- | --- | 32M |
| 4M | 4M | 4M | --- | 48M |
| 4M | 4M | 4M | 4M | 64M |

Bank 2 and Bank 3 are on the memory expansion card.

Remark :

In JAGUARII REV 2.0. The cache memory are support 32K,64K and without internal cache.