STANDARD MICROSYSTEMS CORPORATION

PC AGENT/SNMP

For EtherCard PLUS Family LAN adapters

and TokenCard EliteSeries LAN adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

INTRODUCTION

------------

The PC AGENT/SNMP is a TSR(Terminate but Stay Resident program) which

allows the targeted PC workstation to be remotely managed by a SNMP

manager station(e.g a work station running SMC/EliteView) on the

network through SNMP(Simple Network Management Protocol). The PC

AGENT/SNMP collects and maintains system and statistic information

about the PC.  The SNMP manager can set and retrieve these

information from the PC station with PC AGENT/SNMP loaded.

SYSTEM REQUIREMENTS

-------------------

The PC AGENT/SNMP can run on any XT/AT, EISA, or Micro Channel PC.

An MSDOS or PCDOS operating system is required, version 3.1 or above.

An agent-compatible network driver must be resident in memory.  The

SMC network drivers that are compatible with the PC AGENT/SNMP are :

NDIS Token-Ring driver - SMC8100.DOS (ver 1.00 and above)

NDIS Ethernet driver - SMCMAC.DOS (ver2.00 and above)

ODI Token-Ring driver - SMC8100.COM (ver 1.03 and above)

ODI Ethernet driver - SMC8000.COM (ver 3.03 and above)

IPX Token-Ring driver - IPX.COM (ver 1.00 and above)

The PC AGENT/SNMP TSR will occupy approximately 24K bytes and can be

loaded into either high or low memory.

An IP address must be available if the agent needs to communicate

with SNMP network management stations using TCP/IP protocol.

The PC AGENT/SNMP does not require any network operating system or

protocol stack to be loaded.

INSTALLATION PROCEDURE

----------------------

To load PC AGENT/SNMP, type

PCAGENT  [configuration file]

The parameters in [] are optional.

A configuration file can be specified for the PC AGENT/SNMP. For

details, see CONFIGURATION FILE CONTENTS.

The PC AGENT/SNMP consists of a single executable file, PCAGENT.EXE.

It can be executed at any time once an appropriate IPX, ODI or NDIS

driver is in memory.

For example, the following lines in AUTOEXEC.BAT would be adequate to

load the SMC Ethernet ODI driver(SMC8000) and PC AGENT/SNMP with

PCDATA.CFG as the configuration file:

LSL

SMC8000

PCAGENT PCDATA.CFG

The following lines in AUTOEXEC.BAT would be adequate to load the SMC

Token Ring ODI driver(SMC8100) and PC AGENT/SNMP:

LSL

SMC8100

PCAGENT PCDATA.CFG

The following lines in CONFIG.SYS and AUTOEXEC.BAT would be adequate

to load a Token Ring NDIS driver and PC AGENT/SNMP:

In CONFIG.SYS:

DEVICE=C:\NDIS\PROTOCOL.DOS /I:C:\NDIS

DEVICE=C:\NDIS\SMC8100.DOS

In AUTOEXEC.BAT :

NETBIND

PCAGENT PCDATA.CFG

Note that PC AGENT/SNMP may be executed immediately after the driver

is loaded, as in the examples above, or at any later time.  It may be

convenient to have the PC AGENT/SNMP configuration files for the

various PCs on a network server where they are controlled by a

network administrator.  In this case, for example in a NetWare

environment, the network administrator has allocated an user area on

network drive F: for the various users' configuration files; the

following lines might be in user Kathy's AUTOEXEC.BAT file:

LSL

SMC8000

IPXODI

NETX

LOGIN

PCAGENT F:\USERS\AGTSTUFF\KATHY.CFG

CONFIGURATION FILE CONTENTS

---------------------------

The PC AGENT/SNMP maintains a data base of configuration and

statistical information in its RAM space.  The agent itself discovers

or otherwise controls most of the data.  A few pieces of

configuration information are assigned by the owner of the PC or by

the network administrator, such as the name of the person who uses

the PC and its location.  The configuration file is the mechanism by

which these data are given to the agent.  Each line in the

configuration file specifies the default value of a configuration

entry.  These data are mostly optional.  If they aren't supplied to

the agent, it won't supply them to network management workstations

who query.  One item, the IP address of the agent (smcDosWsIpAdr), must

be given if the agent has to be addressed across the cable by network

management stations which use TCP/IP protocol.

When PC AGENT/SNMP is executed, it first examines its PC,

initializing its data base accordingly.  After that, if the operator

has given it a configuration file to look at, it reads the

configuration file data, initializes those uninitialized data in its

data base and in some cases, overwrites initialized data in the data

base.  For example, the agent may initialize its description of

display type to "EGA/VGA" (generic and non-descript) which the

installer, via the configuration file, might replace with "ATI VGA

Wonder w/512KB" to be more specific.

The syntax of lines in the configuration file is very simple:

DataItemName = DataItemValue

where DataItemName is a data variable name and DataItemValue is the

value the variable should have.  Data variable names which can be set

in the configuration file are listed below.  Variable names are case

sensitive.

An example of a possible configuration file might be:

ComStr = mypassword		     => Read/write password for SNMP

sysName = Sally Jones		     => Person who uses the PC

smcDosWsIpAdr = 129.254.2.3	     => IP address of PC AGENT/SNMP

sysLocation = somewhere on earth    => Location of PC

smcDosWsPcTyp = IBM AT              => PC make and model

smcDosWsTrapDstAdr.1 = 129.254.2.1  => First IP address to send

alerts(traps) to

smcDosWsTrapDstPro.1 = 2	     => Protocol for alerts

(2=TCP/IP)

Required data element :

This data element is required if the agent has to communicate with

SNMP network management stations using TCP/IP protocol.  Otherwise,

it is only optional.

smcDosWsIpAdr

This is the IP address for the PC AGENT/SNMP.  If the PC also has a

TCP/IP stack, the agent may be given the same or a different IP

address than the TCP/IP stack.  It is often convenient to assign the

agent its own IP address to avoid confusion about who is who.  Note

that the agent must be given its IP address in the configuration

file regardless of whether a stack is loaded on the PC with the same

IP address.  Since TCP/IP stacks vary widely in the way their IP

addresses are specified, the agent has no way of discovering the IP

address of such a stack.

Optional data elements :

Data elements which an installer might desire to initialize via the

configuration file.

ComStr

This is the password which SNMP manager uses for read/write

operations.

sysContact

This is usually the person to call for help.  It is initialized by

the agent with SMC's technical support telephone number in the USA.

sysName

This is usually the owner/user of the PC.  It is not initialized by

the agent.

sysLocation

This is usually the physical location of the PC.  It is not

initialized by the agent.

smcDosWsPcTyp

This is the make and model of the PC.  It is not initialized by the

agent.

smcDosWsPcProc

This is the processor type and whether there is a co-processor. The

agent initializes it according to the processor it discovers on the

PC.

smcDosWsPcBios

This is the BIOS manufacturer and version number.  The PC tries to

discern this by scanning for copyright notices in the BIOS area.

smcDosWsPcVideo

This is the type of video adapter on the PC.

smcDosWsTrapDstAdr.1

In a TCP/IP network environment, this is the IP address(in decimal)

of a network management station to which traps(alerts) should be

sent in case an alarm situation occurs.  In an IPX network

environment, this is the IPX network address(in decimal) of the

network management station where traps should be sent.  There is a

maximum of 3 management stations where traps can be sent(Adr.1,

Adr.2, and Adr.3).

smcDosWsTrapDstPro.1

This indicates the protocol in which traps should be sent to the

network management station with IP address or IPX network address

specified by smcDOSWsTrapDstAdr.1

A number is used to specified the protocol used :

2 = TCP/IP(Ethernet & TokenRing)

3 = IPX with Ethernet II frame type(Ethernet ONLY)

4 = IPX with 802.2(LLC) frame type(Ethernet & TokenRing)

5 = IPX with SNAP frame type(Ethernet & TokenRing)

6 = IPX with 802.3 frame type(Ethernet ONLY)

smcDosWsTrapDstAdr.2

This is the second IP address or IPX network address of a network

management station to which traps (alerts) should be sent in case an

alarm situation occurs.

smcDosWsTrapDstPro.2

This indicates the protocol in which traps should be sent to the

network management station with IP address or IPX network address

specified by smcDOSWsTrapDstAdr.2

See smcDosWsTrapDstPro.1 for the number used to specify the

protocol.

smcDosWsTrapDstAdr.3

This is the third IP address or IPX network address of a network

management station to which traps (alerts) should be sent in case an

alarm situation occurs.

smcDosWsTrapDstPro.3

This indicates the protocol in which traps should be sent to the

network management station with IP address or IPX address specified

by smcDOSWsTrapDstAdr.3

See smcDosWsTrapDstPro.1 for the number used to specify the

protocol.

ERROR MESSAGES

--------------

PC AGENT/SNMP was not installed.

Too many parameters in command line.

PC AGENT/SNMP was not installed.

Invalid parameter in command line.

PC AGENT/SNMP was already installed.

PC AGENT/SNMP was not installed.

DOS version too old (below 3.10); need DOS 3.10 or above.

PC AGENT/SNMP was not installed.

Internal error (1) : Call SMC Technical Support.

PC AGENT/SNMP was not installed.

Internal error (2) : Call SMC Technical Support.

PC AGENT/SNMP was not installed.

PC AGENT/SNMP compatible network driver was not loaded.

PC AGENT/SNMP was not installed.

There is no network adapter for the driver.

PC AGENT/SNMP was not installed.

Unable to open configuration file <file name>.

WARNING MESSAGES

----------------

PC AGENT/SNMP Configuration file error :

invalid MIB variable <name>.

Line : < erroneous line in configuration file >

PC AGENT/SNMP Configuration file error :

invalid IP address <the IP address>.

Line : < erroneous line in configuration file >

PC AGENT/SNMP Configuration file error :

invalid numeric value for MIB variable.

Line : < erroneous line in configuration file >

PC AGENT/SNMP Configuration file error :

MIB variable <name> cannot be set from configuration file.

Line : < erroneous line in configuration file >

*****************************************************************

Warning: PC AGENT/SNMP was not able to find a free interrupt vector

for application trap API.  External application programs

will not be able to issue traps.

*****************************************************************

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard

Microsystems Corporation.  Other trademarks mentioned herein belong

to their respective companies.