From: OPHER SZICHMAN

Subj: No #47 BIOS user definition.

---------------------------------------------------------------------------

First, I assume you have an AT class computer (equipped with a CMOS) &

running a DOS Version 4.0+ & that your problematic drive has an IDE

interface.

Now, there are three steps to do this:

1. Select a drive type (in CMOS) closest to your drive's

parameters, noting the number of sectors per cylinder

(usually 17). Then run FDISK and allocate all/some of the

HD space to a DOS Primary Partition so that a MBR is placed

on the HD. Your computer reboots.

2. Use DOS's FORMAT on the drive. Then, Patch the MBR code

& Partition Table to reflect your drive parameters

(explained later). Copy the Boot-Record located

at Sectors_Per_Cylinder + 1 or 1/1/0 (S/H/C, usually sector

18) to absolute sector 2 (the one following the MBR).

Reboot from your HD (remove your DOS diskette in drive A)...

3. You get the 'Non system disk...' message. Place a DOS diskette in

drive A and strike any key. Run DOS's FORMAT on your HD again (format

c:/s - make system disk). If you did everything correctly Format

should show the correct capacity ('Formatting 204 MB...').

Patching the MBR:  The following is a portion of my MBR...

-----------------

cs:7C00 FA             cli                    ; The bootstrap loader loads

cs:7C01 33C0           xor    ax,ax           ; the MBR at 0:7c00

cs:7C03 8ED0           mov    ss,ax

cs:7C05 BC007C         mov    sp,7C00

cs:7C08 8BF4           mov    si,sp           ; si = 7c00, save a byte

cs:7C0A 50             push   ax

cs:7C0B 07             pop    es              ; es = 0

cs:7C0C 50             push   ax

cs:7C0D 1F             pop    ds              ; ds = 0

cs:7C0E FB             sti

cs:7C0F FC             cld

cs:7C10 BF0006         mov    di,0600         ; Setup si, es, ds, di

cs:7C13 B90001         mov    cx,0100         ; & cx to...

cs:7C16 F2A5           repnz movsw            ; move the MBR to 0:0600.

cs:7C18 EAF0060000     jmp    0000:06F0       <- My modification. Originally

.                                               jmp 0000:061D.

.                                               (next instruction)

.

.                                               ----Code to change disk param.

cs:7CF0 BF0003         mov    di,0300

cs:7CF3 BE0401         mov    si,0104         ; Have INT 41h (HD Param.

cs:7CF6 893C           mov    [si],di         ; Table) point to new param.

cs:7CF8 894402         mov    [si+02],ax      ; table.

cs:7CFB BEE006         mov    si,06E0

cs:7CFE B90800         mov    cx,0008

cs:7D01 FC             cld

cs:7D02 F3A5           rep movsw              ; Copy HDPT to 0:0300

;  (safe location)

cs:7D04 B409           mov    ah,09

cs:7D06 B280           mov    dl,80

cs:7D08 CD13           int    13              ; Inform BIOS of new HDPT.

cs:7D0A E910FF         jmp    7C1D            <- Continue with original code.

To make the patching easy you need a disk-editor (Norton Utilities,

PCTOOLS etc.) Use the DE (disk-editor) to view the MBR in hex mode.

If you used DOS 5.0 you should read:

00000000: FA 33 C0 8E D0 BC 00 7C 8B F4 50 07 50 1F FB FC

00000010: BF 00 06 B9 00 01 F2 A5 EA 1D 06 00 00 BE BE 07

^^

Change the pointed byte (1D) to F0. Then change the following:

000000F0: BF 00 03 BE 04 01 89 3C 89 44 02 BE E0 06 B9 08

00000100: 00 FC F3 A5 B4 09 B2 80 CD 13 E9 10 FF

Next at 000000E0 enter 16 bytes where...

000000E0: 00 04 0C 00 00 FF FF 00 08 00 00 00 01 04 22 00

\ /  |         \ /     |            \ /  |

A   B          C      D             E   F

A. Number of cylinders (0400h = 1024)

B. Number of heads (0Ch = 12)

C. Starting write precompensation cylinder - If not known use FFFF

D. Control byte - Bit 3 = 1 if more than eight heads

E. Landing zone - If not known use Number_of_cylinders + 1

F. Number of sectors (22h = 34)

Having done that the only thing left to do is patch the Partition Table:

Enter the partition editor and update the following:

A. System = BIGDOS (06h)

B. Starting Head = 0

Starting Cylinder = 0

Starting Sector = 2

C. Relative Sectors = 1

D. Number of Sectors = The number of sectors allocated to that partition - If

it's all of the HD space then that's:

Sectors_per_cylinder * Heads * Cylinders - 1.

All Done!

If you haven't got a disk editor, the following instructions

are for DEBUG/SYMDEB:

Enter the debugger & assemble the following instructions at CS:0100:

** all numbers are hex.

mov bx,200            ; ES:BX->Location of buffer to place read sectors

mov dx,80             ; Phisical HD number

mov cx,1              ; Starting sector number, S/H/C = 1/0/0

mov ax,201            ; AH=Read sectors, AL=Num. of sectors to read

int 13

int 1                 ; Debugger regains control!

mov bx,400            ; ES:BX->Following MBR

mov dh,1              ; S/H/C = 1/1/0

mov ax,201

int 13                ; Read Boot-Record

xor dh,dh             ; DH=0, S/H/C = 1/0/0

mov ax,302            ; Write 2 sectors: MBR then Boot-Record

int 13

int 1

Press `r<CR>` (<CR>=the enter key) and make sure the

instruction listed is `mov bx,200`. Press `g<CR>`...

Patch the MBR as I explained, noting:

1. Offset of MBR start is DS:0200 (not 00000000).

2. You change a byte's content by typing `e <offset> <value> <value> ...`.

3. The partition table is at DS:03BE.

Again press `r<CR>` & make sure the insruction listed is `mov bx,400`.

Press `g<CR>`... You're done.

Exit the debugger.

��Ŀ

����> Greetings from Opher Szichman.

--- GoldED/P16 2.42.G1121

* Origin: Frontdoor style is not a style, it is a bad habit (2:403/169.3)