Documentation for MultiSearch Text Door

Setting Up a DSZ Configuration File

Written by Robert V. Chambers

Copyright (c) 1996

If  you are running a one or two node board with standard comm ports you might

not  need  to do any configuration in  order  to successfully run DSZ, GSZ, or

FDSZ.  The  door is set up to run  a default parameter set that should work in

most cases. The default command line for DSZ or GSZ is:

port <port num> handshake on sb <path to file>

|        |         |         |       |___ path to selected file

|        |         |         |___________ use YModem (or other protocol)

|        |         |_____________________ handshake on

|        |_______________________________ port number (from drop file)

|________________________________________ port command

FDSZ accepts a smaller list of commands. The default command line for FDSZ is:

<port num> sb <path to file>

|       |        |___________________ path to selected file

|       |____________________________ use YModem (or other protocol)

|____________________________________ port number (from drop file)

Note that the actual program name (ex: dsz) is supplied by the door along with

these  default  command  lines. Remember you  selected  the program type using

option 3 in MULTEDIT.EXE.

If  you are using non-standard port settings  or if you need to add additional

parameters the defaults may not work for you.

If  this  is the case then you will  have to create DSZ.CFG using any Dos text

editor.  This  file will let you fully  specify  most command line options you

need  to  successfully  run DSZ, GSZ, or  FDSZ.  Create  this file in the same

directory  as the other door files. If the  door does not detect that the file

exists  it  will use the default  settings  described above. Otherwise it will

read  DSZ.CFG  for  your  specified  parameters.  As  such  you  MUST  specify

parameters for every comm port your system uses. DSZ.CFG will be read prior to

each download.

The  door will assume that if DSZ.CFG  exists, then YOU are taking control and

that  you will specify each and every comm port independently. It is essential

that you fully describe each comm port you will be using. The door will detect

the  port  that the user is using and  will use either the default settings or

your settings, depending on whether or not DSZ.CFG exists.

Note that you cannot specify send commands (ex: sz) or pathnames that are part

of the complete DSZ command line. This is taken care of internally by the door

depending  on  the  selected protocol. DSZ.CFG  is  used  only to specify port

information for each comm port.  Likewise you cannot specify any commands that

might follow the send command.

The format for DSZ.CFG is:

comm port number

comm port parameters

You must have a two-line pair like this for each comm port you will be using.

If  you  are  specifying  non-standard comm  port  addresses  you MUST use the

"portx"  DSZ command for DSZ or GSZ. If  you are unsure about this please read

your  DSZ documentation. Here is an example of how to assign port 2 to address

2f8, interrupt 3:

2 <-------------------------- port 2 (door will detect user's port)

portx 2f8,3 ha on <---------- base address 2f8,IRQ 3  handshake on

(FDSZ  does  not  support portx. You typically  do  not  need a config file if

running FDSZ.)

For standard comm ports you could specify:

1 <--------------------------- port 1 (door will detect user's port)

port 1 handshake both <------- port 1, handshake both

Again,  you will note that you do  NOT specify the program name, the protocol,

or the path to the file. The door will do this.

-----------------------------------------------------------------------------

NOTE

Only COM1 and COM2 are standard on all PC compatible computers.

COM1 uses 3F8 and IRQ4.  COM2 used 2F8 and IRQ3.

Multiple standards exist for COM3 and above.

-----------------------------------------------------------------------------

DSZ COM3 uses 3E8 and IRQ4.  DSZ COM4 uses 2E8 and IRQ3.

DSZ COM7 uses 3E8 and IRQ5.  DSZ COM8 uses 2E8 and IRQ5.

If  you  wish  to use COM3 or COM4 and  your  computer  is not set up as shown

above, you MUST use the "portx" command to specify the actual base address and

interrupt vector.

Note  that  COM1 and COM3 use the same  IRQ.  You should not have two hardware

devices  that  use  these  ports  at the  same  time  or  problems will almost

certainly  arise, even if both devices are not active. The same applies to the

COM2-COM4 pair. Please read the following excerpt from the DSZ documentation.

This is from the DSZ documentation file:

"     ***********************************************************

Except as noted, all commands and options must be typed in lower case.

DO NOT USE UPPER CASE!!

Case is significant!

***********************************************************

Optional commands or arguments are shown in square brackets [].

When  used (in any combination), the  A, CON, port, speed, handshake,

estimate, and d commands must be used in exactly this order.

dsz  [A]  [CON]  [F] [port N] [speed  S]  [handshake XX] [pxN... ] [z

pxN... ] [D] [d] [restrict] [G] command

Incorrectly  configured serial ports that work with programs that pig

out  both  IRQ3 and IRQ4 will lock up  with  DSZ. If DSZ locks up the

machine  as soon as it tries to send or receive data, this is usually

the  fault. Make sure the comm  port is strapped correctly; COM1 uses

IRQ4,  COM2  uses  IRQ3.  DSZ will lock  up  if  the comm port is not

properly set up.

In  addition, DSZ will not work properly if two devices are connected

to  the same IRQ line, even when  some other programs (that don't use

interrupts as intensively) do work normally on the same machine. Make

sure  no other device uses the same IRQ as DSZ even if that device is

not active.

COM3 and above are not widely standardized. If you have problems with

COM3 or COM4, chances are your "COM3" is non standard."