RUCKUS-DAC Copyright (C)1993 Cornel Huth

Documentation Copyright (C)1993 Cornel Huth

All Rights Reserved.

Version 1.0d.

18-Oct-93/C.

For ordering information, license agreement, and product support

information, see Appendix Z.

Introduction

RUCKUS-DAC is one part of the RUCKUS sound toolkit for DOS programmers. Its

forte is its ability to play digital data in three different formats and

record in two: VOC, WAVE, and MOD. The other part of the RUCKUS sound

toolkit is RUCKUS-MIDI. Its forte is its ability to play standard MIDI data

files. See RUCKUS-MIDI for more.

RUCKUS-DAC currently supports six sound devices: the PC speaker, a DAC on

an LPT port, Disney Sound Source, AdLib MSC, Sound Blaster, and the Sound

Blaster Pro (stereo). Since most sound devices emulate one of the above,

RUCKUS-DAC should handle most devices likely to be found in a PC. RUCKUS-

DAC is implemented to be device-independent. As new sound hardware (with

supporting documentation) becomes available to this programmer, it will be

added. The code you write to play any of the digital formats using RUCKUS

will play on any new device added later without any code changes to your

program. You just supply the new device ID (see InitDac for more).

The RUCKUS toolkit supports most DOS compilers as is. This documentation

describes using RUCKUS with C DOS compilers. Also available is a BASIC-

specific version (BASIC7/QB45/VBDOS/etc.) of this documentation. Other

implementations should be able to get the necessary details using this

documentation. Note that these toolkits will not work with the current or

past Turbo Pascal compilers. This is due to Turbo Pascal's proprietary

linkage methods.

To provide compiler-independence, RUCKUS (and all of my latest toolkits)

implements a simple calling scheme: a single far pointer (segmented) is

passed to RUCKUS. The data structure that is pointed to varies depending on

RUCKUS function. There are 13 structure types ("packs") currently. The

first two data members of each pack are identical in all packs. These are

Func and Stat. The remaining members, if any, vary depending on

requirements. For instance, the LoadDac routine uses, in addition to Func

and Stat, six additional members: FilenamePtr, StartPos, LoadSize, XMMflag,

XMMhandle and LoadPtr. The steps needed to load a digital (LoadDac is for

VOC/WAVE files, LoadMod is for MOD files) data file to memory would be as

follows:

1. Allocate pack. Packs are reusable and can be allocated any way you

want. Here we make a static allocation for simplicity.

struct LoadPack LP;

2. Assign member values:

gets(filename);

LP.Func = LoadDac;

LP.FilenamePtr = filename;

LP.StartPos = 0L;

LP.LoadSize = 0L;

LP.XMMflag = 0;

3. Call RUCKUS:

rez = RUCKDAC(&LP);

4. On return from the call:

if (rez == 0)

// LP.LoadPtr -> address of digital data in memory

// or, if XMMflag then LP.XMMhandle is XMS handle of storage

After the load you can immediately call the PlayDac routine using the

LP.LoadPtr returned from the LoadDac call. You don't need to use the RUCKUS

LoadDac routine; you can perform your own digital data file loads to memory

(VOC and WAV only, MODs must be loaded by RUCKUS-DAC). However, RUCKUS can

load up to 32 separate digital files at one time (and 96 more to XMS) and

performs all the necessary memory allocations and deallocations

automatically. See LoadDac/LoadMod for more.

As you can see, using RUCKUS is easy. If you'd rather use a traditional

calling convention, wrap the RUCKUS routines in a C function and call it as

you normally would, e.g., rez = LoadDacFile(filename,LP) where LoadDacFile

contains the actual code (as shown above).

Now that I have your interest, let me tell you why I've provided this

toolkit for you. Shareware. That's right. Shareware. What I've done is

provide you with a means to evaluate this product completely, with no

upfront obligations. If you can't make use of this product, no problem.

Think of this as the ultimate return policy. You can actually take the

goods home, try it out, break it in. Then, if after a period of no more

than 21 days, you feel that this product is well worth the price I'm

asking, by all means, you should go ahead and fill out the order form and

send it in. If you do not register, then of course, it means the product is

not something you can use. If you can't use it, I don't want your money.

But if it's something you will use, it's only fair that you register. The

PROFESSIONAL registered versions does not have any built-in advertising

screens. The PERSONAL DEVELOPER registered version does display a sign-on

banner once. See Appendix Z. for ordering options and other related

information specific to this product.

The documentation that follows describes each of the function routines as

to use, pack type used, and sample code in C. Also available is a BASIC

version of this toolkit, differing only in example programs and

documentation. For additional source examples see the included X files on

disk.

RUCKUS-DAC Functions.

0. SysInfoDac  ...........................................  5

1. InitDac  ..............................................  6

2. ExitDac  ..............................................  7

3. AtExitDac  ............................................  8

4. LoadDac  ..............................................  9

5. PlayDac  .............................................. 10

6. RecordDac  ............................................ 11

7. StoreDac  ............................................. 13

8. EndDac  ............................................... 14

9. PauseDac  ............................................. 15

10. DeallocDac  ........................................... 16

11 to 19 are not used

20. SetAllDac  ............................................ 17

21. SetVolumeDac  ......................................... 19

22. SetIntRateDac  ........................................ 20

23. SetPriorityDac  ....................................... 21

24 to 27 are not used

28. GetBufferDataDac  ..................................... 22

29. GetBytePosDac  ........................................ 23

30. SetAllSBP  ............................................ 24

31. SetVolMainSBP ......................................... 26

32. SetVolVocSBP  ......................................... 27

33. SetLevelMicSBP  ....................................... 28

34. SetLevelCDSBP   ....................................... 29

35. SetLevelLineSBP  ...................................... 30

36. SetFilterOutSBP  ...................................... 31

37. SetFilterInSBP  ....................................... 32

38. SetSourceSBP  ......................................... 33

39. SetStereoSBP  ......................................... 34

40. SetSpeakerSB  ......................................... 35

41 to 47 are not used

48. GetMixerRegSBP  ....................................... 36

49. GetDacSB .............................................. 37

50. ExitMod  .............................................. 38

51. AtExitMod  ............................................ 39

52. LoadMod  .............................................. 40

53. PlayMod  .............................................. 41

54. EndMod  ............................................... 42

55. PauseMod  ............................................. 43

56. SetIntRateMod ......................................... 44

57. SetSpeedMod ........................................... 45

58. SetVolumeMod  ......................................... 46

59. SetPositionMod ........................................ 47

60. SetStereoMod .......................................... 48

61. SetFastMod ............................................ 49

Appendix A. Tips and Tricks ........................... 50

Appendix B. Pack Structure ............................ 51

Appendix C. Compiler, Linker and Call Use ............. 55

Appendix D. RUCKUS-DAC Data Area ...................... 56

Appendix Z. Ordering Information, License Agreement

and Product Support........................ 58

SysInfoDac          Func: 0        Pack: SysInfoPack

Before doing any other RUCKUS function, you should call SysInfoDac to

determine what sound devices are available. RUCKUS-DAC currently supports

the following devices:

0. PC speaker

1. LPT-DAC device on a printer port, commonly home-built

2. Disney Sound Source, a printer port DAC device

3. AdLib Music Synthesizer Card

4. Sound Blaster (DSP version 1.03 and up)

5. Sound Blaster Pro

6. UltraSound*

*The UltraSound (GUS) will be supported in an upcoming release. PAS16

native support soon after.

Since many current sound cards are Sound Blaster compatible, RUCKUS-DAC

supports somewhere around 95% of all sound devices in active use on PC-

compatible computers.

SysInfoDac scans for and detects all devices. In the case of the DAC on the

printer port (LPT-DAC), it can only detect if a printer port is available.

All other sound cards are detected by their manufacturer's documented

methods, and should be 100% reliable. Information returned includes the

device's base port address, IRQ line, DMA channel, and capabilities.

Since the SB Pro is SB-compatible, and since the SB is AdLib-compatible,

these lower devices are also detected in these cards.

Sample Use:

struct SysInfoPack SIP;

SIP.Func = SysInfoDac;   //SysInfoDac is a CONST in RUCKDAC.BI

rez = RUCKDAC(&SIP);     //call to RUCKUS library code

if (rezt == 0) {

printf("CPU type is %u/%u\n", SIP.CPU, SIP.MHz);

if (SIP.SD[5].Device != 0)

printf("SB PRO at port %x\n", SIP.SD[5].Port);

if (SIP.SD[4].Device != 0)

printf("SB 1.5 at port %x\n", SIP.SD[4].Port);

:

printf("PC speaker always available\n");

}

5

InitDac             Func: 1        Pack: InitPack

Before you can use a sound device, you must inform RUCKUS about it. The

InitDac routine sets up default sets of data that RUCKUS uses, or that you

can later change. You cannot operate a device until you have called this

routine. You should not InitDac more than one sound device at a time. If

you want to use a different device, first ExitDac then InitDac again.

The information that InitDac requires is available from the SysInfoDac

routine. InitDac returns with information on its data section address and

size, and the address of the ExitDac routine (in case AtExitDac doesn't

work for your esoteric compiler or run-time library).

InitDac is used for both RUCKUS-DAC and the RUCKUS-DAC mod extension code.

There is a special feature available for the PC speaker and the AdLib

devices: 2x upsampling. To use 2x upsampling, use the I/O port as returned

by SysInfoDac for IP.IOport. To use standard (in some cases downsampling)

play for the PC speaker and AdLib (recommended for play of samples with

rates above 8kHz) use an IP.IOport=0. For an example of 2x upsampling see

the MODAC1 documentation. Upsampling is available only for VOC and WAVE

playback.

Sample Use:

struct InitPack IP;

IP.Func = InitDac;       // CONST in RUCKDAC.BI

IP.DeviceID = device;    // see SysInfoDac for device numbers

IP.IOport = port;        // and other info needed by InitDac

IP.IRQline = IRQ;

IP.DMAch = DMA;

rez = RUCKDAC(&IP);

if (rez == 0)

// device initialized okay

6

ExitDac             Func: 2        Pack: XitPack

When using RUCKUS, certain aspects of the computer are controlled by the

RUCKUS code. To have RUCKUS return full control back to the computer and

operating system, ExitDac must be called. To ensure that this happens, even

after an abnormal termination (such as an untrapped divide by zero), RUCKUS

provides a routine that ensures that ExitDac is always called before your

application ends. See AtExitDac for more.

Once a sound device has been initialized with InitDac, you must call

ExitDac before you can switch to another device with InitDac. Calling

ExitDac releases any memory allocations made while the previous sound

device was active (allocations made by LoadDac, LoadMod, or RecordDac).

Memory allocations that your program has made on its own are not affected.

Tip: If you call InitDac with one device, load a file, then call InitDac

again with another device, the memory allocated by the load is not

released. This allows you to play data from previous load operation(s) on

all the RUCKUS-supported devices. To force the release of a previous memory

allocation, see DeallocDac.

Sample Use:

struct XitPack XP;

XP.Func = ExitDac;

rez = RUCKDAC(&XP);

if (rez != 0)

// error

7

AtExitDac           Func: 3        Pack: XitPack

To ensure that all machine control is returned to the operating system and

that all RUCKUS-made memory allocations are released before your

application program ends, AtExitDac should be used. This routine takes

advantage of most compiler's _atexit enregistration routine. This _atexit

compiler run-time library routine allows for up to 32 different calls to be

made just prior to it (the compiler's run-time code) returning to the

operating system (program termination). Most of the time, you can make all

the necessary shutdowns that your program requires in your app's code.

However, abnormal terminations -- terminations that can cause a bypass of

your own shutdown code -- may mean that when the operating systems regains

control, certain machine operations are not in a stable state. The

AtExitDac routine ensures that ExitDac is called no matter what (in most

all cases, except where the program locks up, in which case the session

needs to be reset anyway).

Note: AtExitDac should be called only once in your program's execution.

Graphically, your program executes like this:

DOS C>program.exe

|

Compiler startup code (performs basic setup)

|

+----->+

| Your program starts, ends

+<-----+

|

rest of startup code, _atexit registered routines

|

DOS C>_

Caution: In order for the _atexit registered routines to be called by your

compilers run-time, you must not allow you program to exit via non-standard

ways. Calling INT21/4C is on example (you should never do this from

compiled code, anyway). Note that some compilers' run-time code bypasses

_atexit when Ctrl-C or Ctrl-Break is used. If this is the case, you must

trap these and perform an ExitDac before returning to DOS.

The startup code is basically a program that sets up your program to run,

calls your program, regains control when your program ends, then performs

the necessary shutdown actions required, including calling all routines

registered with _atexit.

Sample Use:

struct XitPack XP;

XP.Func = AtExitDac;

rez = RUCKDAC(&XP);

if (rez !=0)

// error (more than 32 calls made to _atexit)

8

LoadDac             Func: 4        Pack: LoadPack

Though not required, you can have RUCKUS load a digital file into memory

for you. It makes the necessary memory allocations and loads the data into

conventional or XMS memory. You can load up to 32 separate digital data

files to conventional memory at one time, and up to 96 more into XMS.

RUCKUS takes care of the memory allocation details.

LoadDac can also be instructed to start loading at any particular byte in a

file for as many bytes as you specify. This would be useful, for example,

if you have a single file made up of several VOC or WAVE files appended to

form one large data file.

Sample Use:

struct LoadPack LP;

LP.Func = LoadDac;

LP.FilenamePtr = filename;

LP.StartPos = 0L;         // start load at first byte of file

LP.LoadSize = 0L;         // have RUCKUS load all of it

LP.XMMflag = 0;           // -1 if loading into XMS

rez = RUCKDAC(&LP);

if (rez == 0)

// okay load

Note that even though loading into XMS memory, RUCKUS-DAC still needs some

conventional memory to use as a buffer. After the load to XMS, the low

memory is no longer required.

9

PlayDac             Func: 5        Pack: PlaybackPack

Once the digital data is in either conventional or XMS memory, you can play

it to any of the RUCKUS-supported devices. XMS memory, however, requires

DMA playback mode; this mode is supported in the Sound Blasters only. Also,

for STEREO playback, DMA mode is required (SB Pro only).

In addition to DMA-mode playback, you can instruct RUCKUS to play in the

background or foreground. Playing in the background allows your computer

program to regain control immediately after play has begun. If you choose

foreground play, control does not return to your program until the end of

the data has been played, or, until CTRL-ALT is pressed.

All RUCKUS-supported devices can be played in the background. Only the

Sound Blasters can be played using DMA. DMA playback is similar to

background play in that control is returned to your program immediately,

but it is also much more efficient since DMA playback uses no CPU time.

Background play, on the other hand, can require anywhere from 1 to 100% of

available CPU time, depending on what playback rate has been selected (see

SetIntRateDac).

Since it's possible to load several data files into memory at one time, to

play a particular data file requires that you specify its start address or,

if loaded to XMS memory, its XMS handle. Given the code sequence example in

LoadDac, the following PlayDac code takes up from there:

struct PlaybackPack PBP;

PBP.Func = PlayDac;

PBP.Mode = 2;       // mode 0=foreground, 1=background, 2=DMA

PBP.XMMhandle = 0;  // 0=conv memory data follow, 1+=XMS data follows

'If conventional memory used, then the following pointer is the

'segment:offset start address of the VOC/WAVE data to play.

'

'If XMS memory used, the following pointer is the 32-bit offset within the

'XMS handle (PBP.XMMhandle) that the data starts. If you just load one data

'file to one XMM handle, then these will always=0. However, if you're short

'on available XMM handles, you could load several data files into a single,

'huge, XMM allocation. In order to do this, you must do your own data

'loads since LoadDac loads one file per XMM handle.

'

PBP.LoadPtr = LP.LoadPtr;

'Note: the PBP.BufferSize is used by PlayMod only. Also, you may want/need

'to perform pre-Play tasks. See SetAllDac/SetAllSBP.

rez = RUCKDAC(&PBP);

if (rez == 0)

// if DMA or background mode then the digital data is playing now,

// but if foreground mode, the play has ended by the time we get here

10

RecordDac           Func: 6        Pack: RecordPack

The Sound Blasters can record as well as play digital data. As with play,

you can record directly into conventional or XMS memory. You can let RUCKUS

make the memory allocations, or you can allocate your own memory and record

into that. With the SB Pro, you can also record in stereo.

With the Sound Blaster Pro you can select among three input sources: mic,

line, and CD. The earlier Sound Blaster has one input, the mic. A quality

microphone can make a big difference. For other-than-mic input on the SB,

you can pipe the headphone output of a CD player into the mic input. Your

results may vary, but this is better than mic'ing a loudspeaker. For

anything other than mic input, the SB Pro is recommended. Its recording

quality is very good since it allows sampling rates up to 45.5kHz in mono,

and 22.75kHz in stereo. The SB allows recording sampling rates up to 12kHz

(rated to 13kHz record and 23kHz playback). The sampling size is 8 bits.

NOTE: TO RECORD IN STEREO AT A GIVEN SAMPLE RATE, YOU MUST SPECIFY TWICE

THE RATE DESIRED. FOR EXAMPLE, TO RECORD OFF A CD DECK CONNECTED TO

THE LINE INPUT OF THE PRO, AT A SAMPLE RATE OF 22.5kHz, YOU MUST

SET THE SP.SetIntRate=44100.

11

Sample Use:

struct RecordPack RP;

struct XitPack XP;

EP.Func = EndDac;         // preload EP.Func here for space reasons

RP.Func = RecordDac;

RP.SampleRate = 23000;

/* To record into XMM and have RUCKUS make the allocation, set RP.XMMhandle

to -1. If you made your own XMS allocation, set this to your handle.

To load into conventional memory and have RUCKUS make the allocation,

set RP.RecordPtr to NULL. If you made your own allocation, set this

to your far segmented pointer. */

RP.XMMhandle = 0;         // use conventional memory if this is 0

RP.RecordPtrOff = 0;      // have RUCKUS make the memory allocation

RP.RecordPtrSeg = 0;      // if this is 0 (and if RP.XMMhandle=0)

/* By using the DACDATA.MemDOS or .MemXMM, we can record as many bytes

as we have available. Below we reserve 5K (more for XMS) for misc use, as

you should, too. This is the max bytes to record; we can also stop

at any point before this. */

RP.RecordBytes = (DACDATA.MemDOS - 5) * 1024;

RP.StereoFlag = 0;

rez = RUCKDAC(&RP);

if (rez == 0)  {

// we are recording right now, using DMA, so, while we could do just

// about anything we wanted, in this example, we just wait until the

// record is over

do

;

while (DACDATA.End == 0);

rez2 = RUCKDAC(&XP);   // stop recording (XP defined/assigned above)

}

/* The above loop won't be exited until all the bytes requested have been

recorded. The number of bytes recorded (not counting any file headers added

by StoreDac) is in DACDATA.RecordLen.

Now you can save the data by using StoreDac, or play it using PlayDac. On

return from the RECORD, RP.RecordPtr is set to the segment:offset of the

first byte recorded in conventional memory, or if using XMS memory,

RP.XMMhandle is set to the XMS handle. */

12

StoreDac            Func: 7        Pack: SaveDataPack

Once digital data has been loaded or recorded in memory, you can store it

do disk in either VOC or WAVE format. If you want, you can load a file in

one format and save it in the other with just a few lines of code (shown

below).

Sample Use:

struct SaveDataPack SDP;

SDP.Func = StoreDac;

SDP.FilenamePtr= filename;

// the LP pointer below is from (say) a just-loaded VOC file (using

// LoadDac), and we want to convert and store it to WAVE format

SDP.DataPtr = LP.LoadPtr // segment:offset of conventional memory

// or 0 if storing from an XMS handle

SDP.FileType = 2;        // 1=VOC, 2=WAVE

SDP.XMMhandle = 0;       // 0 if DOS data, else XMS handle

rez = RUCKDAC(&SDP);

if (rez == 0)

// data stored to filename$

else

//file save error in stat

/* Caution: Be sure to account for the additional memory required once data

has been stored to a file. StoreDac builds the appropriate VOC or WAV

header and this adds to the total memory required to reload. I suggest that

you do not record so much data as to not be able to load it back in once

saved to VOC or WAVE format. */

13

EndDac              Func: 8        Pack: XitPack

EndDac is used to end (stop) play or record on command. Once PlayDac has

started, it continues to play in the background until either the end of the

digital data has been reached, or you call this routine. In the case of a

record, play continues until the number of bytes requested has been

recorded, or this routine is called.

Sample Use:

struct XitPack XP;

// LoadDac and PlayDac, or RecordDac, have already been called

// see those routines for more information

XP.Func = EndDac;     // if play or record is active this call

rez = RUCKDAC(&XP);   // ends it

// See RecordDac on getting the number of bytes actually recorded

14

PauseDac            Func: 9        Pack: PausePack

PauseDac allows you to temporarily pause and then restart a playback.

Sample Use:

struct PausePack PP;

PP.Func = PauseDac;

PP.Pause = 1;        // non-zero pauses, zero restarts

rez = RUCKDAC(&PP);

15

DeallocDac          Func: 10       Pack: DeallocPack

If you have RUCKUS load digital data files, which is most likely the case,

it makes memory allocations from the operating system memory pool. RUCKUS

tracks up to 32 consecutive DOS loads, and up to 96 loads to XMS (provided

enough memory and XMS handles). These should be higher than you'll ever

likely use at one time. DeallocDac "unloads" files that RUCKUS has loaded,

thus releasing the memory that they used.

As an example, let's say that you want to start up your program with 5

digital data files in XMS memory. You have 5MB of XMS available. Each data

file is 1M in size. To load a new data file after these 5, you will need to

unload one of the 5 already loaded. DeallocDac does that for you. The

following example loads a single data file to XMS then immediately unloads

it.

DeallocDac is not used for releasing LoadMod allocations.

Sample Use:

struct LoadPack LP;

struct DeallocPack DP;

LP.Func = LoadDac;

LP.FilenamePtr = filename;

LP.StartPos = 0L;

LP.LoadSize = 0L;

LP.XMMflag = 1;        // non-zero means use XMS for load

rez = RUCKDAC(&LP);

if (rez == 0) {

DP.Func = DeallocDac;

DP.HandSeg = LP.XMMhandle; // XMM handle returned by RUCKDAC(LP)

DP.TypeFlag = LP.XMMflag;  // 0=DOS allocation, 1=XMS

rez = RUCKDAC(&DP);     // if this were a DOS allocation, DP.HandSeg

// would be set to LP.LoadPtrSeg only since

// _FP_OFF(LP.LoadPtr)==0 as returned by LoadDac

}

if (rez != 0)

// file load error or deallocate error in rez

16

SetAllDac           Func: 20       Pack: SetPack

This routine sets the overall playback volume, interrupt rate, and RUCKUS

priority in one call.

The playback volume affects VOC or WAVE output for non-DMA mode, only. In

other words, it's for controlling the volume on the PC speaker, LPT-DAC,

Disney Sound Source, and AdLib only. To control the playback volume of the

Sound Blaster, you must operate it under non-DMA mode. To control the

playback of the Sound Blaster Pro, a separate set of RUCKUS routines are

used. See the SetAllSBP routine for more.

The default volume is set to 16. Minimum is 0, maximum is 127. For a linear

approximation of this volume control, 16 is about half-volume, 8 about 1/4,

64 about 3/4, and 127 full volume. A volume other than 16 may

cause a slowdown in performance so 16 should be used as much as possible.

Note: SETTING THE VOLUME TO 0 TURNS OFF THE SOUND BLASTER AND PRO'S

SPEAKER (REQUIRED BEFORE RECORDING). ANY OTHER SETTING TURNS

THE SPEAKER CONNECTION ON.

The interrupt rate sets the playback rate of background play for non-DMA

mode. For example, let's say you load a file that was recorded with a

sample rate of 8000Hz (this information is stored with the VOC or WAVE).

That is the value to which you set the interrupt rate. Easy enough. But

let's say that it was recorded at 44,100Hz and in stereo. Normally, this

data file couldn't be played except with high-end sound cards. What you can

do is set the interrupt rate to 8000Hz and have RUCKUS-DAC perform on-the-

fly downsampling so that you can play this file on any of the supported

devices (in non-DMA mode).

In addition to downsampling, RUCKUS-DAC can upsample with the PC Speaker

and AdLib cards (the two that need it the most). Let's take that 8000Hz

sample rate file again. Upsampling performs on-the-fly adjustments to the

data so that it sounds as if the file sample rate were twice the original.

This results in a much cleaner sound. There is a drawback: much more CPU

time is needed at this high rate, and it should only be used on powerful

CPUs. To use the upsampling capabilities of RUCKUS-DAC, set the interrupt

rate to the original sample rate. There is also a special method used in

the initialization of the device. See InitDac for more. You can experiment

with the down- and upsampling of RUCKUS-DAC by using the MODAC.EXE program.

When using non-DMA mode, you can select from three priority levels. The

default priority=1. This level allows the system to run as normal, even

though the computer's ticker has been sped up some thousand times. Priority

level 2 hogs all background time for the RUCKUS-DAC playback. The

computer's clock is not updated. Priority level 0 gives top priority to

other computer events. This level may result in a warbly sound. Priority

only affects non-DMA VOC/WAVE output. Use the default level (1).

17

The default interrupt rate is 10989. If you are on a slow machine, this may

be too high for non-DMA operations. The following table shows the min, max,

and nominal interrupt rate for the supported devices. In all cases, higher

rates require more CPU power, except in DMA mode. Nominal values based on a

386/25 CPU.

Device   MinRate   MaxRate  Nominal  Comments

------   -------   -------  -------  ------------------------------------

PCSKP0      5000     18000     8000  rates < 16000 may be noisy

PCSPK1      5000      9000     8000  effective rate=rate*2

LPTDAC      5000     23000    15000  not tested

DISNEY      5000      7000     7000  max usable rate is 7000Hz

ADLIB0      5000     12000     8000  AdLib very slow as digital device

ADLIB1      5000      6000     5500  effective rate=rate*2

SB          5000     23000    15000  DMA mode is 5-23000, record to 12kHz

SB PRO      5000     23000    15000  DMA mode is 5-45500

Sample Use:

struct SetPack SP;

SP.Func = SetAllDac;

SP.Volume = 16;       // any other value slows down performance

SP.IntRate = 8000;

SP.Priority = 1;

rez = RUCKDAC(&SP);

18

SetVolumeDac        Func: 21       Pack: SetPack

SetVolumeDac does two things. First, it controls the non-DMA mode output

volume. Second, it turns the SB's speaker on and off; the SB speaker must

be off, and remain off, while recording. Under normal use, the SB speaker

is turned on and off by RUCKUS automatically.

Because of the extra calculations required when the volume is set to

anything other than the default (16), it's recommended that the volume be

changed only when necessary (especially on slow machines).

This routine controls the output volume on all devices in non-DMA mode. In

DMA-mode, the SB's volume is not controllable. The SB PRO, however, has a

large set of mixer controls that can be controlled via RUCKUS-DAC. See

SetAllSBP for more.

Sample Use:

struct SetPack SP;

SP.Func = SetVolumeDac;

SP.Volume = 127;          // set to max volume

rez = RUCKDAC(&SP);

if (rez != 0)

// you probably didn't set SP.Func correctly

19

SetIntRateDac       Func: 22       Pack: SetPack

SetIntRateDac, with respect to the -DAC section of RUCKUS, controls the

playback rate of non-DMA output.

After loading a file, that file's sample rate is available in the DAC data

section. You may choose to have the digital data file played back at that

rate, or, you may choose to downsample the original rate. Downsampling is

useful since it allows slower computers to play data files originally

sampled at rates too high to be played back -- RUCKUS can play a 44.1kHz

stereo VOC or WAVE file back through the PC speaker, or any of the other

supported devices. Note that the actual downsampled rate is not always

exactly what you specify, but a best-fit approximation.

As an example, let's suppose you have a VOC file originally sampled at

22050Hz, mono format. While this can easily be played on the SBs using DMA-

mode playback, non-DMA mode requires a very powerful CPU to handle that

high a rate. After loading using LoadDac, the 22050 sample rate, and its

mono format, is determined by reading the DAC data section. To play back

this file through PCSPKR0, say, at 11kHz, you'd set SP.IntRate=11025.

With the PC speaker and the AdLib, there are two different output modes.

The first either plays at the original rate, or is downsampled to a lower

rate (as specified by you). The second mode upsamples. By upsampling,

RUCKUS can output a much cleaner sound than playing at the original sample

rate. However, this is useful only for samples that were originally at a

low sample rate. You would not want to upsample a VOC file that was sampled

at 22kHz, since that would demand too much CPU power, plus, very little

benefit would be achieved. The best use of upsampling is on data that was

originally sampled at, say, 8kHz or less. In order to properly upsample,

you need to specify a SP.IntRate equal to the original sample rate (this

can be found in the DAC data section immediately after a LoadDac).

As mentioned, upsample mode is available on the speaker and AdLib devices

only. To specify this mode, set the IP.IOport to the physical port when

initializing the device (InitDac). In the PC speaker's case, IP.IOport=0x42

would activate up-sample mode. Using IP.IOport=0 would have the regular

mode used. For the AdLib, specify IP.IOport=0x388. As with the PC speaker,

setting IP.IOport=0 uses the regular AdLib play mode.

Sample Use:

struct SetPack SP;

SP.Func = SetIntRateDac;

SP.IntRate = 11025;

rez = RUCKDAC(&SP);

20

SetPriorityDac      Func: 23       Pack: SetPack

When operating in non-DMA mode playback of VOC or WAVE data, RUCKUS

reprograms the system timer. Normally, this timer runs at the familiar 18.2

times per second rate. When you play back at, say, a SP.IntRate=10000, this

is the new rate that the system timer is set to operate at. So, instead of

the timer interrupting the CPU (via IRQ0) 18.2/sec, it interrupts at

10,000/sec.

The routine allows you to control the priority given to other events while

the timer is operating at this accelerated pace. The default priority is 1.

At the priority, the playback code is called at the desired rate, and,

given a 10kHz rate, every 550th time the playback code is executed, the

original timer-dependant code is executed. This allows the system clock,

and any other system in the computer dependant on the clock, to be properly

maintained. On each 550th call, RUCKUS first performs all tasks that needs

to be done, then it calls the original system code. In most cases, if not

all, this priority works best.

Priority 0 is exactly like priority 1 except that control is released as

soon as the RUCKUS code starts. The permits other tasks to interrupt

RUCKUS. This may cause a warbly-type sound, but permits, say, high-speed

communications code to operate in the background with missing a beat.

Priority 2 causes RUCKUS to use (hog) all background processing time for

itself. The system clock is not maintained. Use this priority only if the

other two are causing problems (doubtful) or if you want to get the maximum

sound performance out of RUCKUS and don't care if the system clock is

maintained.

Sample Use:

struct SetPack SP;

SP.Func = SetPriorityDac;

SP.Priority = 1;   // 1 is the default so no need to call unless changed

rez = RUCKDAC(&SP);

21

GetBufferDataDac    Func: 28       Pack: GetDataPack

Used to get a specified number of bytes from the current digital data path.

This routine is currently under development and should not be used.

22

GetBytePosDac       Func: 29       Pack: GetDataPack

Used to determine where in the current digital data path processing is

occurring.

Sample Use:

struct GetDataPack GDP;

GDP.Func = GetBytePosDac;

rez = RUCKDAC(&GDP);

if (rez == 0)

printf("Curr pos relative start of data is %ul", GDP.BytePos);

23

SetAllSBP           Func: 30       Pack: SetProPack

This routine allows you to control the mixer facility of the Sound Blaster

Pro including main volume, VOC volume, mic level, CD level, line level,

output filter, input source selection, record filters, input source, and

stereo/mono record mode.

During initialization of the Sound Blaster Pro (InitDac), the main and VOC

volumes are set to maximum (15) for both left and right channels, the three

input levels are set to their minimums (1), all filters are set to off,

mode is set to mono record, and input selection is set to mic.

NOTE: PLAYBACK OF VOC AND WAVE STEREO DATA FILES IS AUTOMATIC. THE STEREO

MODE SETTING IS FOR RECORDING ONLY. TO PLAY STEREO FILES IN MONO

ON THE SB PRO, USE THE SOUND BLASTER (DEVICE 4) INSTEAD.

The main volume (left and right, as are all mixer controls except mic)

adjusts both the digital and synthesizer outputs of the PRO (the

synthesizer, or FM section, is not used by RUCKUS-DAC, but is in RUCKUS-

MIDI). In normal use the main volume channels would be set to 15, the

maximum.

The VOC volume adjusts the volume of the digital output channels. Anything

played through the SB PRO's digital channels can be adjusted with this. As

with the main volume, the VOC volume is normally set to 15, the maximum.

The SB PRO provides for three inputs. Mic, CD, and Line. Each of these have

separate level adjustments. The mic input level is the odd one out. Unlike

the other two, the mic level does not adjust the mic input level, but

rather the mic output level. Whenever you record via the mic input line at

the back of the card, the input level is controlled by an AGC, automatic

gain control. The AGC acts as a real-time level adjuster -- high signal

levels (loud sounds) have the AGC automatically decrease the mic input

sensitivity -- low signal levels (quiet sounds) have it automatically

increase the mic input sensitivity. The actions of an AGC has its drawbacks

in reduced dynamic range and perhaps a noticeable "breathing" of the sound

on playback. Breathing is where the background noise increases as the level

of the sound decreases. However, often the reduced dynamic range makes for

a better final results, especially when using a mic/SB combination.

The CD input level adjusts the level of the CD-ROM device. No testing has

been done with this. It controls the input level from the proprietary CD-

ROM drive that CLabs distributes for the SB PRO, or one which is compatible

(and is hooked up to the SB PRO's CD connector). Any CD should be able to

plug into the CD audio connector of the SB PRO (see the SB PRO docs) and so

have its level controllable via RUCKUS-DAC.

The Line input level adjusts the input sensitivity. Normally, when

recording from this input, the Line level should be set to 15, the maximum.

For preamps that have an unusually high line out level, reducing the PRO's

line input level may produce a better recording.

The output filter can be used to reduce the noise at the mid and high end

of the digital data being played. Normally, this filter is not needed.

24

The recording filters are very useful and should be used whenever the

recording sample rate is below 18kHz and the source being record has

frequencies above 9kHz. There are two filters to choose from: one has a

3.2kHz cutoff, the other 8.8kHz.

Whenever you are recording from a source that will include frequencies

higher than 3.2kHz, and you have the PRO recording at a sample rate under

6.4kHz, you should select the 3.2kHz filter. This will attenuate

frequencies higher than 3.2kHz and should help produce a cleaner recording.

Along the same lines, if you are recording frequencies higher than 8.8kHz,

and are recording using a sample rate less than 17.6kHz, you should use the

8.8kHz filter. Note that filter use as described above is not a rule, just

a guideline. You may find that using no filters is best.

With the SB PRO you have 3 input sources to record from: the mic, a CD-ROM

drive attached to the SB PRO (requires special cable), or a line-level

input. The mic allows mono recording only. The CD-ROM facility controls

only a CD-ROM drive hooked up to the SB PRO. The line input can be

connected directly to a preamplifier's line out jacks.

The SB PRO allows you to record in STEREO from the CD and LINE input

sources; mic is always mono. When recording in stereo, the sample rate you

record at must be set to twice the desired rate. For example, if you want

to record from the LINE input at 22.05kHz, in stereo, set the recording

rate to twice 22,050, or 44,100. The SB PRO can sample up to a 45,500Hz

rate (which, if a stereo recording, is an effective rate of 22,750Hz). See

RecordDac for more.

Sample Use:

struct SetProPack SPP;  // be careful not to type SBP

SPP.Func = SetAllSBP;

SPP.Volume = 0x0F0F;    // max out volumes

SPP.VolVoc = 0x0F0F;

SPP.VolMic = 0;         // minimized mic input level

SPP.CD = 0;             // actual min for SB Pro is 1

SPP.LI = 0;

SPP.FilterOut = 0;      // 0=off filter, 1=on (output filter)

SPP.FilterIn = 0;       // 0=off filter, 1=3.2kHz, 2=8.8kHz (record filter)

SPP.SourceIn = 0;       // 0=mic, 1=CD, 2=line

SPP.StereoIn = 0;       // record mode: 0=mono, 1=stereo

rez = RUCKDAC(&SPP);

25

SetVolMainSBP       Func: 31       Pack: SetProPack

The main volume (left and right) adjusts both the digital and synthesizer

outputs of the PRO; it is the overall volume setting. Each of these outputs

also has its own volume control. In normal use, the main volume channels

would be set to 15, the maximum.

The range of volume is 1 to 15, in steps of 2 (1,3,5,7,9,11,13,15). To set

the left channel main volume, multiply the volume desired by 256. To set

the right channel, just add its value. For example, to set the main volume

to maximum (15, or 0F hex), multiply 15*256 and add 15. In hex, this value

is 0F0F (decimal 3855, but decimal is much too awkward). If you know

beforehand that you want the main volume at maximum, you can forego the

multiplication and simply use 0F0F, as in the example below.

NOTE: Since you are the programmer and you have the documentation, do not

depend on RUCKUS correcting out-of-value parameters. If you use

parameters that are not within specifications, the results are

undefined.

Sample Use:

struct SetProPack SPP;

SPP.Func = SetVolMainSBP;

SPP.Volume = 0x0F0F;

rez = RUCKDAC(&SPP);

26

SetVolVocSBP        Func: 32       Pack: SetProPack

The VOC volume (left and right) adjusts the digital output volume of the

PRO. The range of volume is 1 to 15, in steps of 2 (1,3,5,7,9,11,13,15). To

set the left channel VOC volume, multiply the volume desired by 256. To set

the right channel, just add its value. For example, to set the VOC volume

to maximum (15, or 0F hex), multiply 15*256 and add 15. In hex, this value

is 0F0F (decimal 3855, but decimal is much too awkward). If you know

beforehand that you want the VOC volume at maximum, you can forego the

multiplication and simply use 0F0F, as in the example below.

Sample Use:

struct SetProPack SPP;

SPP.Func = SetVolMainSBP;

SPP.VolVoc = 0x0F0F;

rez = RUCKDAC(&SPP);

27

SetLevelMicSBP      Func: 33       Pack: SetProPack

This routine does not actually adjust the mic input level, but rather the

mic output level. Whenever you record via the mic input line at the back of

the card, the input level is controlled by an AGC, automatic gain control.

The AGC acts as a real-time level adjuster -- high signal levels (loud

sounds) have the AGC automatically decrease the mic input sensitivity --

low signal levels (quiet sounds) have it automatically increase the mic

input sensitivity. The actions of an AGC has its drawbacks in reduced

dynamic range and perhaps a noticeable "breathing" of the sound on

playback. Breathing is where the background noise increases as the level of

the sound decreases. However, often the reduced dynamic range makes for a

better final results, especially when using a mic/SB combination.

By setting the mic input level, you can use the computer as a public

address system. To do this, select mic as the input source, set the mic

input level to maximum, and turn on the SB speaker. To avoid feedback, keep

the mic away from the loudspeakers.

The input level range is 1 to 7, in steps of 2 (1,3,5,7). This setting has

no effect on recording level (see above).

Sample Use:

struct SetProPack SPP;

SPP.Func = SetVolMicSBP;

SPP.VolVoc = 0x07;

rez = RUCKDAC(&SPP);

28

SetLevelCDSBP       Func: 34       Pack: SetProPack

The CD input level (left and right) adjusts the input level of the attached

CD-ROM drive. This has not been tested. For its use, see SetLevelLineSBP.

29

SetLevelLineSBP     Func: 35       Pack: SetProPack

The line level (left and right) adjusts the analog input level of the PRO.

The range of volume is 1 to 15, in steps of 2 (1,3,5,7,9,11,13,15). To set

the left channel line level, multiply the volume desired by 256. To set the

right channel, just add its value. For example, to set the line level to

maximum sensitivity (15, or 0F hex), multiply 15*256 and add 15. In hex,

this value is 0F0F (decimal 3855, but decimal is much too awkward). If you

know beforehand that you want the line level at maximum, you can forego the

multiplication and simply use 0F0F, as in the example below.

Sample Use:

struct SetProPack SPP;

SPP.Func = SetVolMainSBP;

SPP.VolLI = 0x0F0F;

stat = RUCKDAC(&SPP);

30

SetFilterOutSBP     Func: 36       Pack: SetProPack

The output filter of the PRO can be used to reduce the noise at the mid and

high end of the digital data being played. Normally, this filter is not

needed. Recordings made at a very low sample rate, say, under 6kHz, may

sound better with this output filter on.

The digital output filter, and also the analog recording filter, is set off

by default.

Sample Use:

struct SetProPack SPP;

SPP.Func = SetFilterOutSBP;

SPP.FilterOut = 1;          // enable output filter

rez = RUCKDAC(&SPP);

31

SetFilterInSBP      Func: 37       Pack: SetProPack

The recording filters are very useful and should be used whenever the

recording sample rate is below 18kHz and the source being record has

frequencies above 9kHz. There are two filters to choose from: one has a

3.2kHz cutoff, the other 8.8kHz.

Whenever you are recording from a source that will include frequencies

higher than 3.2kHz, and you have the PRO recording at a sample rate under

6.4kHz, you should select the 3.2kHz filter. This will attenuate

frequencies higher than 3.2kHz and should help produce a cleaner recording.

Along the same lines, if you are recording frequencies higher than 8.8kHz,

and are recording using a sample rate less than 17.6kHz, you should use the

8.8kHz filter. Note that filter use as described above is not a rule, just

a guideline. You may find that using no filters is best.

The input filter is off by default.

Sample Use:

struct SetProPack SPP;

SPP.Func = SetFilterInSBP;

SPP.FilterIn = 1;       // 0=no filter, 1=3.2kHz cutoff, 2=8.8kHz cutoff

rez = RUCKDAC(&SPP);

32

SetSourceSBP        Func: 38       Pack: SetProPack

With the SB PRO you have three input sources to record from: the mic, a CD-

ROM drive attached to the SB PRO (requires special cable), or a line-level

input. The mic allows mono recording only. The CD-ROM facility controls

only a CD-ROM drive hooked up to the SB PRO. The line input can be

connected directly to a preamplifier's line out jacks.

Recording with the mic requires only that it be selected. The mic input

level does not control the recording level, but rather the PA output. See

SetAllSBP for more.

Recording from the connected CD-ROM drive has not been tested. If you can

control the CD, you can record from it using RUCKUS.

Recording from the line-level input is simple. Just hook your tape-out

jacks from your preamplifier to the cable supplied with the SB PRO. This

cable, a 1/8-inch mini stereo to 2 RCA phono jacks is the same one that

comes with the original Sound Blaster. You'll probably need more cable

unless you have your preamp nearby.

When recording from either the CD or line, you must set the source's input

level. Normally, setting it to maximum (15) is best. If you have

particularly high line output levels, you may need to set this lower. As

with all mixer settings, settings are 1 to 15 in steps of 2. The mic is 1

to 7, by 2.

Sample Use:

struct SetProPack SPP;

SPP.Func = SetSourceSBP;

SPP.SourceIn = 2;         // set input source to line input

rez = RUCKDAC(&SPP);

33

SetStereoSBP        Func: 39       Pack: SetProPack

The SB PRO allows you to record in STEREO from the CD and LINE input

sources; mic is always mono. When recording in stereo, the sample rate you

record at must be set to twice the desired rate. For example, if you want

to record from the LINE input at 22.05kHz, in stereo, set the recording

rate to twice 22,050, or 44,100. The SB PRO can sample up to a 45,500Hz

rate (which, if a stereo recording, is an effective rate of 22,750Hz). See

RecordDac for more.

You must set the mode to stereo or mono before starting the actual record.

Sample Use:

struct SetProPack SPP;

SPP.Func = SetStereoSBP;

SPP.StereoIn = 1;         // set to stereo record

rez = RUCKDAC(&SPP);

34

SetSpeakerSB        Func: 40       Pack: SetPack

The routine activates or deactivates the Sound Blasters' output. When

recording it must be off. When playing back, it must be on. When starting a

recording, RUCKUS automatically turns the speaker off; when playing back it

turns the speaker on. For those occasions when you want to control the

speaker connection, use this routine.

Note that the generic SetVolumeDac also controls the SB's speaker

connection. If the speaker is currently on and you set the volume to 0, the

SB speaker is turned off. And, of the speaker is currently off, and you set

the volume to a non-zero value, the speaker is turned on. This routine lets

you control the connection without changing the generic volume setting.

Sample Use:

struct SetPack SP;

SP.Func = SetSpeakerSB;

SP.Volume = 0;           // use the volume member of SP to control speaker

rez = RUCKDAC(&SP);      // to turn the speaker on, set SP.Volume = 1

// THIS DOES NOT ALTER THE VOLUME

35

GetMixerRegSBP      Func: 48       Pack: GetDataPack

Get a SB PRO mixer port-value.

This routine is under development and should not be used.

36

GetDacSB            Func: 49       Pack: GetDataPack

This routine gets a byte from the Sound Blasters data channel. This routine

should not be called while the Sound Blaster is playing or record. It can

be used for level tests.

Before using this routine on the PRO, first select the input device (mic,

CD, or line) and then perform a 64-byte record from that device. This

because the input device is not selected until you actually record from it.

Unlike the other RUCKDAC routines, the return value is not the error code,

but rather the data byte itself. To check for an error, use GDP.stat (as

you can in all RUCKUS functions).

Sample Use:

struct GetDataPack GDP;

GDP.Func = GetDacSB;

data = RUCKDAC(&GDP);

if (GDP.stat != 0)

// error

37

ExitMod             Func: 50       Pack: XitPack

When using RUCKUS, certain aspects of the computer are controlled by the

RUCKUS code. To have RUCKUS return full control back to the computer and

operating system, ExitMod must be called. To ensure that this happens, even

after an abnormal termination (such as an untrapped divide by zero), RUCKUS

also provides a routine that ensures that ExitMod is always called before

your application ends and returns to the operating system. See AtExitMod

for more.

Once you have loaded a mod file via LoadMod, or performed any mod function,

you must call ExitMod before you end you program and return to DOS. Calling

ExitMod releases any memory allocations made, and also performs other

system cleanup.

Sample Use:

struct XitPack XP;

XP.Func = ExitMod;

rez = RUCKDAC(&XP);

38

AtExitMod           Func: 51       Pack: XitPack

To ensure that all machine control is returned to the operating system and

that all RUCKUS-made memory allocations are released, before your

application program ends, AtExitMod should be used (if you have used the

mod extensions to RUCKUS-DAC). This routine takes advantage of most

compiler's _atexit enregistration routine. This _atexit compiler run-time

library routine allows for up to 32 different calls to be made just prior

to it (the compiler's run-time code) returning to the operating system

(program termination). Most of the time, you can make all the necessary

shutdowns that your program requires in your app's code. However, abnormal

terminations--terminations that can cause a bypass of your own shutdown

code, may mean that when the operating systems regains control, certain

machine operations are not in a stable state. The AtExitMod routine ensures

that ExitMod is called no matter what (in most all cases, except where the

program locks up, in which case the session needs to be reset anyway).

Graphically, your program executes like this:

DOS C>program.exe

|

Compiler startup code (performs basic setup)

|

+----->+

| Your program starts, ends

+<-----+

|

rest of startup code, _atexit registered routines

|

DOS C>_

Caution: In order for the _atexit registered routines to be called by your

compilers run-time, you must not allow you program to exit via non-standard

ways. Calling INT21/4C is on example (you should never do this from

compiled code, anyway).

The startup code is basically a program that sets up your program to run,

calls your program, regains control when your program ends, then performs

the necessary shutdown actions required, including calling all routines

registered with _atexit. See AtExitDac for an important addition.

Sample Use:

struct XitPack XP;

XP.Func = AtExitMod;

rez = RUCKDAC(&XP);

if (rez != 0)

puts("_atexit is full (more than 32 calls made to _atexit)");

39

LoadMod             Func: 52       Pack: LoadPack

It is required that RUCKUS load mod files into memory. RUCKUS makes the

necessary memory allocations and loads the data into conventional memory,

with UMB blocks used if available and controlled by DOS (via DOS=,UMB in

CONFIG.SYS). Only one mod file may be in memory at any one time. To unload

the mod file you must execute an ExitMod call.

LoadMod can be instructed to start loading at any particular byte in a

file. This would be useful, for example, if you have a single file made up

of several mod files, appended to form one large data file.

Sample Use:

struct LoadPack LP;

LP.Func = LoadMod;

LP.FilenamePtr = filename;

LP.StartPos = 0L;  // start load at first byte of file

LP.LoadSize = 0L;  // have RUCKUS load all of it

LP.XMMflag = 0;    // this must be set to zero

rez = RUCKDAC(&LP);

40

PlayMod             Func: 53       Pack: PlayBackPack

Once the mod file has been loaded, you can play it to any of the RUCKUS-

supported devices. There are 4 modes that can be used for playback; non-DMA

foreground, non-DMA background, DMA foreground, and DMA background. The DMA

modes require the Sound Blasters. Also, for STEREO mod playback, DMA mode

is required (SB Pro only).

Playing in the background allows your computer program to regain control

immediately after play has begun. If you choose foreground play, control

does not return to your program until the end of the data has been played,

or, until CTRL-ALT is pressed. In all cases, mod file playback is best done

on a fast computer, though excellent results can be gotten from 286-class

machines. For slow machines, DMA foreground allows for best sound, since

you can set the mod process rate higher than you can if you choose a

background mode.

Before play, you should set up other parameters by calling the other Set

mod routines (SetIntRateMod, SetStereoMod, and SetFastMod).

NOTE: SOME MACHINES ARE NOT ABLE TO HANDLE BACKGROUND DMA MODE AT HIGH

RATES, EVEN ON 486-CLASS MACHINES. HOWEVER, BOTH MY 486/33 AND

MY 286/16 WORK WELL IN THIS MODE. THE 486/33 CAN PROCESS MOD DATA

AT RATES UP TO THE PRO'S MAX, 45,500Hz. THE 286/16 CAN PROCESS UP

TO 16kHz IN THIS MODE AND 22kHz IN DMA FOREGROUND. A 286/6 HAS

BEEN TESTED TO 8kHz (DMA FOREGROUND) AND PERFORMS WELL. A 12MHz

DID 16kHz. ON SLOWER MACHINES, USING A LARGER DMA BUFFER SIZE

AND A HIGHER DMAX: VALUE ALLOWS FOR HIGHER RATES TO BE ACHIEVED.

IT IS POSSIBLE TO LOCK UP A MACHINE IF TOO HIGH A RATE IS ATTEMPTED.

struct PlaybackPack PBP;

PBP.Func = PlayMod;

PBP.Mode = 3;      // mode 0=foreground, 1=background, 2=DMA FG, 3=DMA BG

PBP.XMMhandle = 0; // this must be set to 0

// PBP.LoadPtr is not used by PlayMod

PBP.BufferSize = 4096;// buffer size is bytes per buffer (2 buffers used)

// minimum size is 2048, maximum is 65520

// buffers allocated only if DMA play (mode=2 or 3)

rez = RUCKDAC(&PBP);

if (rez == 0)

// if background mode then the mod data is playing now, but

// if foreground mode, the play has ended by the time we get here

else

// play error (could be not enough memory for PBP.BufferSize)

41

EndMod              Func: 54       Pack: XitPack

EndMod is used to end (stop) play on command. Once PlayMod has started, it

continues to play in the background until either the end of the mod data

has been reached, or you call this routine.

Sample Use:

struct XitPack XP;

// LoadMod and PlayMod have already been called

// see those routines for more information

XP.Func = EndMod;     // if play is active this call

rez = RUCKDAC(&XP);   // ends it

42

PauseMod            Func: 55       Pack: PausePack

PauseMod allows you to temporarily pause and then restart a playback.

Sample Use:

struct PausePack PP;

PP.Func = PauseMod;

PP.Pause = 1;        // non-zero pauses, zero restarts

rez = RUCKDAC(&PP);

43

SetIntRateMod       Func: 56       Pack: SetModPack

SetIntRateMod, with respect to the mod extensions of RUCKUS, controls the

mod data processing rate, which in turn, is the playback rate (the opposite

of sample rate).

A process rate of 22kHz should be considered a practical maximum. Higher

process rates are possible (up to 45.5kHz with the PRO) but the improvement

in sound quality over 22kHz is negligible (to me). If the machine is

capable, a rate of at least 16kHz should be used. At this rate, sound is

reasonably good sounding. Rates below 16kHz will most likely incur a high-

pitched whistling noise that can be very annoying. Using the output filter

of the SB PRO helps. The whistle is totally absent from the output of the

Disney Sound Source (which internally plays at 7kHz regardless of the input

stream rate). For PC speaker output, rates above 18kHz are not recommended.

Best performance for the PC speaker is at 16kHz. The AdLib, being a very

slow device when made to play digital data, has an effective top-end of

about 13kHz.

When using the SB PRO in stereo mode, the mod process rate must not exceed

45.5kHz/2, or 22.75kHz. This is a limitation of the Sound Blaster. In mono,

you can run up to 45.5kHz if you have the muscle to do so (typically

requires a 486, and a DMAX: setting of at least 4). Buffer sizes above 4K

matter most on slower machines.

Adjusting the mod processing while playing is permitted, though it's

recommended to do so only when not.

Sample Use:

struct SetModPack SMP;

SMP.Func = SetIntRateMod;

SMP.IntRate = 22000;

rez = RUCKDAC(&SMP);

44

SetSpeedMod         Func: 57       Pack: SetModPack

Temporarily increases or decreases the playback speed of the mod file

playing. The basic speed of a mod, with regard to this routine, is 6.

However, at any particular point in the mod file this can change.

The speed range is 1 to 15, with 1 resulting in fast play, and 15 resulting

in a very slow play.

Sample Use:

struct SetModPack SMP;

SMP.Func = SetSpeedMod;

SMP.Speed = 4;          // increase play by 33% over norm

rez = RUCKDAC(&SMP);

45

SetVolumeMod        Func: 58       Pack: SetModPack

SetVolumeMod controls each of the four mod digital output channels.

Normally they would all be set to maximum (255). In order to adjust channel

volume, FastMod mode must be off.

Sample Use:

struct SetModPack SMP;

SMP.Func = SetVolumeMod;

SMP.VolCh1 = 255;

SMP.VolCh2 = 255;

SMP.VolCh3 = 255;

SMP.VolCh4 = 255;

rez = RUCKDAC(&SMP);

46

SetPositionMod      Func: 59       Pack: SetModPack

Each mod file is composed of several tracks, known as patterns. Each

pattern contains 64*4 notes (64 notes per channel). A mod file can contain

up to 63 separate patterns, and can play up to 128 patterns in total

(though any pattern can be made to repeat). SetPositionMod lets you move to

the start of any pattern to be played (up to the number of patterns to

play). The first pattern is pattern number 0. The last possible patter

number is 127.

Sample Use:

struct SetModPack SMP;

SMP.Func = SetPositionMod;

SMP.Position = 0;          // restart play from the beginning

rez = RUCKDAC(&SMP);

47

SetStereoMod        Func: 60       Pack: SetModPack

Mod data files contain four channels of information. In mono playback mode,

all four of these are combined to form a single digital output stream. In

stereo mode (SB PRO only), you can select from three different combine

modes. You can increase this to nine stereo combinations by setting the

high bit of SMP.Stereo. If set, the left/right channels are reversed.

Sample Use:

struct SetModPack SMP;

SMP.Func = SetPositionMod;

SMP.Stereo = 1;          // 0=mono

rez = RUCKDAC(&SMP);     // 1=LEFT:ch1+ch3  RIGHT:ch2+ch4

// 2=LEFT:ch1+ch4  RIGHT:ch2+ch3

// 3=LEFT:ch1+ch2  RIGHT:ch3+ch4

// &H8000 + mode then LEFT <-> RIGHT, e.g.,

// &H8001=LEFT:ch2+ch4  RIGHT:ch1+ch3

48

SetFastMod          Func: 61       Pack: SetModPack

Mod files require lost of CPU power and lots of CPU attention. The most

often executed code performs 4 BPTR IMUL instructions every tick of a

process clock (10,000 times/sec if that is the process rate selected).

Multiply on the 8088 is a very time consuming operation. If an 8088-based

computer is to play mod files, these multiplies must be sped up. SetFastMod

performs this task by implementing an alternate method. Note that with

FastMod mode enable, some mod files may sound odd.

The SliceAdj instructs the DMA buffer fill code to perform more than one

mix per interrupt. By increasing this number you reduce the interrupt-

latency time wasted (since more mod data is processed per interrupt) but

you also decrease the amount of time given to other tasks (tasks other than

mod playback). Fast CPUs have no problem keeping the DMA buffers full, so

even a large SliceAdj will have little of any impact on these machines.

However, on slow machines where the DMA buffers are constantly needing to

be filled (where neither DMA buffer is ever filled) a SliceAdj above 1 may

be desirable. Typically, though, the default of 1 works fine.

Sample Use:

struct SetModPack SMP;

SMP.Func = SetFastMod;

SMP.FastMode = -1;      // 0=normal, 1=fast mode enabled, -1 to skip

SMP.SliceAdj = 32;      // DMA buffer fill count (1-4096, default=1)

rez = RUCKDAC(&SMP);

49

Appendix A. Tips and Tricks.

See the RUCKUS-MIDI documenation.

50

Appendix B. Pack Structure.

#pragma pack(1)

int __far __pascal RUCKDAC(void __far *datapack);

#define SysInfoDac      0

#define InitDac         1

#define ExitDac         2

#define AtExitDac       3

#define LoadDac         4

#define PlayDac         5

#define RecordDac       6

#define StoreDac        7

#define EndDac          8

#define PauseDac        9

#define DeallocDac      10

#define SetAllDac       20

#define SetVolumeDac    21

#define SetIntRateDac   22

#define SetPriorityDac  23

#define GetBufferDataDac 28

#define GetBytePosDac   29

#define SetAllSBP       30

#define SetVolMainSBP   31

#define SetVolVocSBP    32

#define SetLevelMicSBP  33

#define SetLevelCDSBP   34

#define SetLevelLineSBP 35

#define SetFilterOutSBP 36

#define SetFilterInSBP  37

#define SetSourceSBP    38

#define SetStereoSBP    39

#define SetSpeakerSB    40

#define GetMixerRegSBP  48

#define GetDacSB        49

#define ExitMod         50

#define AtExitMod       51

#define LoadMod         52

#define PlayMod         53

#define EndMod          54

#define PauseMod        55

#define SetIntRateMod   56

#define SetSpeedMod     57

#define SetVolumeMod    58

#define SetPositionMod  59

#define SetStereoMod    60

#define SetFastMod      61

/* current highest function is 61 */

51

struct DeallocPack {    /* DP */

unsigned Func;

int Stat;

unsigned HandSeg;      /* RUCKUS allocates either XMM handle/DOS para */

unsigned TypeFlag;     /* 0=DOS para, 1=XMS handle */

}; /* 8 */

struct GetDataPack {    /* GDP */

unsigned Func;

int Stat;

unsigned long BytePos; /* current byte relative base ptr */

char __far *BufferPtr; /* far pointer to buffer to fill with data */

long StartPos;         /* start get at this offset relative BufferPtr */

unsigned BufferCnt;    /* bytes to fill (2-65520) */

unsigned MixerReg;     /* SBPro mixer register to get */

}; /* 20 */

struct InitPack {       /* IP */

unsigned Func;

int Stat;

unsigned DeviceID;     /* 0=SPKR,1=LPTDAC,2=DSS,4=SB,5=SBPro */

unsigned IOport;

unsigned IRQline;

unsigned DMAch;

unsigned Flags;        /* see Appendix D. */

void __far *InfoPtr;     /* ret:far ptr to dac info */

void __far *DacExitPtr;  /* ret:far ptr to dac's ExitDac routine */

void __far *ModExitPtr;  /* ret:far ptr to mod's ExitMod routine */

}; /* 26 */

struct LoadPack {       /* LP */

unsigned Func;

int Stat;

void __far *FilenamePtr;/* far ptr to filenameZ to load */

unsigned long StartPos; /* offset into file to start load at */

unsigned long LoadSize; /* number of bytes to load (or 0 for autosize) */

int XMMflag;            /* if <> 0 use XMS for load */

int XMMhandle;          /* ret:XMS handle, or */

void __far *LoadPtr;    /* ret:DOS seg:offset (offset always 0) */

}; /* 24 */

struct PlaybackPack {   /* PBP */

unsigned Func;

int Stat;

unsigned Mode;         /* mode (0=int FG,1=BG,2=DMA,3=DMA+BG for mod) */

unsigned XMMhandle;    /* if <> 0 this XMM handle used regardless */

void __far *LoadPtr;   /* seg:off to start of data to play */

unsigned BufferSize;   /* size of DMA buffer for mod playback */

}; /* 14 */

52

struct PausePack {      /* PP */

unsigned Func;

int Stat;

unsigned Pause;        /* 0=unpause else pause */

}; /* 6 */

struct RecordPack {     /* RP */

unsigned Func;

int Stat;

unsigned SampleRate;

int XMMhandle;         /* -1 auto XMS (ret here) else use this handle */

void __far *RecordPtr; /* seg:off of buffer to store (0 for auto-store) */

unsigned long RecordBytes; /* bytes to record */

unsigned StereoFlag;   /* stereo flag */

}; /* 18 */

struct SaveDataPack {   /* SDP */

unsigned Func;

int Stat;

void __far *FilenamePtr; /* far ptr to filenameZ to save */

void __far *DataPtr;   /* pointer to start of data to save */

unsigned FileType;     /* 1=VOC,2=WAV */

unsigned XMMhandle;    /* XMS handle of data to save (0 if DOS data) */

}; /* 16 */

struct SetPack {        /* SP */

unsigned Func;

int Stat;              /* (if volume=0 SB speaker off'ed else on'ed) */

unsigned Volume;       /* volume (left ch=MSB,right=LSB) (0-127,0-127) */

unsigned IntRate;      /* playback interrupt rate (5000-23000) */

unsigned Priority;     /* priority level (0-2, default=1) */

}; /* 10 */

struct SetModPack {     /* SMP */

unsigned Func;

int Stat;

unsigned VolCh1;       /* channel volume (0-255) */

unsigned VolCh2;       /* channel volumes adjustments made only */

unsigned VolCh3;       /*  if FastMode=0 */

unsigned VolCh4;

unsigned Stereo;       /* playback mode (0=mono,stereo 1,2,3) */

int FastMode;          /* fast playback (0=normal,1 fast,-1 skip) */

unsigned IntRate;      /* playback interrupt rate (5000-45500) */

unsigned Position;     /* pattern list position (0-patterns to play) */

unsigned Speed;        /* temp playback speed (1-15,default=6,15=slow) */

unsigned SliceAdj;     /* slice adj (1-4096,default=1) set via FastMod */

}; /* 24 */

53

struct SetProPack {     /* SPP */

unsigned Func;

int Stat;

unsigned Volume;       /* volume (low=right;0-15, high byte=left;0-15) */

unsigned VolVoc;

unsigned VolMic;       /* (mono only, 0-7) input level */

unsigned VolCD;        /* input level (0-15,0-15) */

unsigned VolLI;        /* input level (0-15,0-15) */

unsigned FilterOut;    /* 0=filter off, 1=filter on */

unsigned FilterIn;     /* 0=filter off, 1=3.2kHz, 2=8.8kHz */

unsigned SourceIn;     /* 0=mic,1=CD,2=line */

unsigned StereoIn;     /* 0=mono,1=stereo record */

}; /* 22 */

struct XitPack {        /* XP */

unsigned Func;

int Stat;

}; /* 4 */

struct SysDev {         /* SD (used by SysInfoPack below) */

int Device;            /* =1 device available */

unsigned Port;

unsigned IRQ;

unsigned DMA;

unsigned Flags;        /* bit4=MIDI/3=XMS/2=DMA/1=REC/0=PLAY */

}; /* 10 */

struct SysInfoPack {    /* SIP */

unsigned Func;

int Stat;

unsigned CPU;          /* CPU class (88,286,386,486) */

unsigned CPUmode;      /* 0=real mode,1=PM,2=PM w/paging */

unsigned MHz;          /* approx speed (5,20,33) */

struct SysDev SD[6];

}; /* 70 */

54

Appendix C. Compiler, Linker, and Call Use.

Consult your development system's documentation on the use of external

library files. For call use, see the example X*.c files.

55

Appendix D. RUCKUS-DAC Data Area.

To access the DAC data area in your code:

extern struct DacDataArea pascal DACDATA;

:

printf("DACDATA DeviceID is %u", DACDATA.DeviceID);

/* dac data area structure */

#pragma pack(1)  /* ALL Ruckus data structures MUST be byte-aligned */

struct DacDataArea {

unsigned DeviceID;     /* 0   ;device ID */

unsigned IOport;       /* 2   ;port of device */

unsigned IRQ;          /* 4   ;IRQ of device */

unsigned DMA;          /* 6   ;DMA of device */

unsigned Flags;        /* 8   ;bit0=1 use DOS UMB memory */

/*     ;bit1-3 reserved */

/*     ;bit4=1 force SBPro if SB15 (DSPtype=3) */

/*     ;       (but DSPversion remains same) */

/*     ;bit5=1 force XMS2 to be used */

/*     ;bit6=1 force dev speaker on until exit */

/*     ;bit7=1 force passed parms to be used */

/*     ;       Following values are ret bits */

/*     ;bit8=1 paging in force, no UMBs */

/*     ;bit9-15 reserved */

unsigned End;          /* 10  ;=1 end of play (dac, not mod) */

unsigned Pause;        /* 12  ;=1 pause play */

unsigned EndOfMod;     /* 14  ;=1 end of mod play */

/*     (when dac@end AND dac@endmod   */

/*     ;both are 1 then play is done) */

unsigned MemDOS;       /* 16  ;DOS memory available (in K) */

unsigned MemXMM;       /* 18  ;XMS memory available (in K) */

unsigned Type;         /* 20  ;digital file type (1=VOC,2=WAV,3=MOD) */

unsigned MemUsed;      /* 22  ;memory used for last file load (in K) */

unsigned SampleRate;   /* 24  ;sample rate currently playing */

unsigned Stereo;       /* 26  ;stereo playback (data is stereo) */

unsigned long VocLen;  /* 28  ;length of voc block (only current block) */

void __far *VocPtrPtr; /* 32  ;pointer to pointer->current data */

unsigned long RecordLen;/*36  ;length of recorded data */

}; /* 40 */

#pragma pack()

56

/*

To access the MOD data area in your code:

extern struct ModDataArea pascal MODDATA;

:

printf("MODDATA Type is %u", MODDATA.Type);

*/

#pragma pack(1)  /* ALL Ruckus data structures MUST be byte-aligned */

struct ModDataArea {

void near *chInfoPtr;/* -2  ;near ptr to channel info (not listed) */

unsigned Type;         /*  0  ;mod type (15 or 31 samples) */

unsigned Samples;      /*  2  ;number of instrument samples in mod */

unsigned HeaderSeg;    /*  4  ;header segment (aka sample info) */

unsigned PatternSeg;   /*  6  ;pats' segment (1 to pats2play 1K pats) */

unsigned SampleSeg[31];/*  8 (+62) ;list of sample segments */

unsigned Stereo;       /* 70  ;=1 then play stereo (only SBPro can set) */

unsigned FastMode;     /* 72  ;=1 then channel volume adjusts disabled */

unsigned PatListPos;   /* 74  ;byte pos within mod@patterList (0-127) */

unsigned NotePatPos;   /* 76  ;note pos within pattern 1K area (0-1008) */

/*     ;pattern#=patternList(patListPos) */

/*     ;note is at offset (pat#*1024)+notePatPos */

/*     ;-- in segment mod@patternSeg */

unsigned HiPattern;    /* 78  ;highest pattern # to play/+1!/ (0-63) */

unsigned Pats2play;    /* 80  ;pats to play (direct DOS file load here) */

char PatternList[128]; /* 82  (+128) ;pattern positions (to here) */

char MKoverflow[6];    /*210 (+6)   ;overflow for 31-instr patpos read */

unsigned MemUsed;      /*216 ;DOS mem needed by mod file loaded (paras) */

unsigned long VS;      /*218 ;times VS handler entered (50Hz rate) */

}; /* 222 */

#pragma pack()

57

Appendix Z. Ordering Information, License Agreement and Product Support.

To order you must you the order form included with the distribution files.

Its filename is !ORDER.FRM. Orders made without this form may be delayed.

There are two RUCKUS packages available. The PERSONAL DEVELOPER version is

for the hobby-programmer while the PROFESSIONAL version is for the

professional programmer.

The PERSONAL DEVELOPER version is for persons that are not creating

programs for distribution to others. With the PERSONAL DEVELOPER license

you may not distribute any programs you create with RUCKUS. In addition, a

sign-on banner is issued once, displaying the RUCKUS copyright and license

restriction.

The PROFESSIONAL version has no distribution restrictions on end-user

programs you create with RUCKUS. The PROFESSIONAL license provides you with

the right to create all the end-user programs royalty-free. You also have

direct access to the latest version of RUCKUS free-of-charge by way of my

support BBS and the RUCKUS Developer's Conference there. No sign-on banner

is issued.

58

License Agreement

Before using this software you must agree to the following:

1. You are not allowed to operate more than one (1) copy of this software

package at one time per license. This means that if you have 10 programmers

that COULD possibly use the RUCKUS library at the same time, you must also

have ten (10) RUCKUS licenses.

2. You are not allowed to distribute non-executable code containing RUCKUS

code. This means that you are not allowed to redistribute RUCKUS code as

another .LIB, for example. Also, if RUCKUS code is to be contained in a

Dynamic Link Library (DLL) then it must be part of a stand-alone product.

This means that you cannot provide a .DLL containing RUCKUS code if that

.DLL is to be used as a programming library for other programmers. If you

wish to distribute non-executable code containing RUCKUS code you must

obtain written permission from the author.

3. This license grants you the right to use the RUCKUS library code on a

royalty-free basis, except when the license is the PERSONAL DEVELOPER, in

which case you may not distribute any program in which RUCKUS has been

used.

4. RUCKUS is owned by the author, Cornel Huth, and is protected by United

States copyright laws and international treaty provisions. You are not

allowed to make copies of this software except for archival purposes.

5. You may not rent or lease RUCKUS. You may not transfer this license

without the written permission of the author. If this software is an update

or upgrade, you may not sell or give away previous versions.

6. You may not reverse engineer, decompile, or disassemble this software.

7. There are no expressed or implied warranties with this software.

8. All liabilities in the use of this software rest with the user.

9. U.S. Government Restricted Rights. This software is provided with

restricted rights. Use, duplication, or disclosure by the Government is

subject to restrictions as set forth in subparagraph (c)(1)(ii) of the

Rights in Technical Data and Computer Software clause at 52.227-7013.

Manufacturer is Cornel Huth/6402 Ingram Rd/San Antonio, TX 78238.

This agreement is governed by the laws of the state of Texas.

59

Product Support

BBS support telephone: 1(210)684-8065. V.32bis (1200-14.4k bps)

Hours: Monday through Friday, 5pm to 9am next morning

Weekend hours are 1pm to 9am next morning.

All times USA Central [-0600/-0500]

E-mail:

Internet: chuth@lonestar.utsa.edu -or-

cornel.huth@LChance.sat.tx.us -or-

cornel.huth@p8.f800.n387.z1.fidonet.org

Fidonet: 1:387/800.8

PROFESSIONAL version licensees have _free_ access to all future RUCKUS

updates AND upgrades via the RUCKUS Developer's Conference on The Fortieth

Floor BBS (1(210)684-8065). PERSONAL DEVELOPER licensees have restricted

access to in-version maintenance updates at no charge.

End of the RUCKUS-DAC DOCUMENT. See also the RUCKUS-MIDI DOCUMENT.