Documentation for IFC routines.

-------------------------------

Source file(s)	: ifc.c

Include file(s)	: ifc.h

Library			: tools.lib

The routines contained in IFC.C are designed to simplify reading from, and

writing to, the parallel ports in order to use them for simple interfacing

circuits.

===============================================================================

Function		int IFCset(int portnum)

Prototype in	ifc.h

Sets the current port to portnum. Portnum can range from 0 to 3, with 0

being LPT1, and 3 being LPT4.

Returns			FALSE if an invalid printer port is specified, TRUE otherwise.

-------------------------------------------------------------------------------

Function		int16 IFCout(uint8 val)

Prototype in	ifc.h

Outputs the 8 bits of val to the current port.

Returns			the value ouput, or -1 if the current port is invalid.

-------------------------------------------------------------------------------

Function		int16 IFCin()

Prototype in	ifc.h

Reads 8 bits of data from the current port.

Returns			the value read, or -1 if the current port is invalid.

===============================================================================

General notes :

---------------

Beginning at location 0040:0008 is a block of 8 bytes. These are arranged as

4 words, 1 for each of the ports LPT1 to LPT4. If this word is 0, the corres-

ponding port is invalid, otherwise the word contains the base I/O address of

the port. Starting from this base addresss are 3 locations of interest.

At [base+0] is an 8-bit latch leading to pins 2-9 on the connector. There is

also a tri-state buffer reverse-connected across this latch, so that reading

this address will return the current states of pins 2-9. Unfortunately, that

does not mean these pins can be used for input, as the latch is not open-

collector. However, this feature is useful for verifying that written data has

actually made it to the hardware.

At [base+1] is an I/O chip, of which bits 3-7 are useful. This chip can

only be used for inputting data. Bits 3-6 will read as 0 if the appropriate

pin is at GND relative to pin 25, or 1 if the pin is at +5v. However, bit 7

is inverted - It will read 0 if pin 11 is at +5v and 1 if it's at GND.

At [base+2] is another chip, of which bits 0-3 can be read or written. This

chip is a set of open-collector output transistors, with tri-state buffers

reverse-connected to enable reading. Bits 0, 1 and 3 are inverted. A 1 in the

bit position will produce GND on the appropriate pin, while a 0 will produce

a floating (+5v) value. Bit 2 will produce +5v if it's 1, or a floating GND

if it's 0.

This gives a total of 17 bits which can be played with, as follows :

Offset	bit		Pin		Direction

0		0		2		Output

0		1		3		Output

0		2		4		Output

0		3		5		Output

0		4		6		Output

0		5		7		Output

0		6		8		Output

0		7		9		Output

1		3		15		Input

1		4		13		Input

1		5		12		Input

1		6		10		Input

1		7		11		Input		Inverted

2		0		 1		Either		Inverted

2		1		14		Either		Inverted

2		2		16		Either

2		3		17		Either		Inverted

As written, my functions use the first 8 bits (Offset 1, 3-7 and Offset 2, 0-2)

as input pins. This gives us a total of 8 inputs and 8 outputs (from Offset 0).

I figured this was a good balance, and it shouldn't be too hard to write some

routines for any other configuration. Some possibly useful configurations

would be :

12 outputs, 5 inputs,

8 outputs,  9 inputs, or

9 outputs,  8 inputs.

WARNING : You should be careful not to change any bits in these locations

other than the necessary ones. I can't remember what they do. Also, Bits 0-3

of Offset 2 terminate at open-collector ouputs on the chip. If you're going

to use these for inputs, you need to make sure you don't short them to +5v

while they are latched at Gnd. However, the other way is quite okay. The safest

bet is to only connect up gear which shorts them to ground, or leaves them Open

circuit. They have pullup resistors internal to them, and if left floating will

quite happily show a '1'.This will prevent damage to the chip, then all you

have to remember is to write them correct value to each of these bits before

doing any reading from them. This makes sure the outputs are not S/C to ground

by the chip itself. If this doesn't make sense, the following suggested

connection diagram might help.

PC               |                Your Gear

|

+5v              |

|               |

Z resistor      |

|---------------------- SWITCH

|/               |         |

Bit 0-3   ----|                |         |

|\               |        Gnd

|               |

|               |

Gnd              |

|

Here's the pin to port/bit assignments.

Pin		IFCallout	IFCallin	IFCout	IFCin	Port/Bit	Direction

-----------------------------------------------------------------------------

1		   8		   0				  0		   2/0		Both

2		   0					  0				   0/0		Out

3		   1					  1				   0/1		Out

4		   2					  2				   0/2		Out

5		   3					  3				   0/3		Out

6		   4					  4				   0/4		Out

7		   5					  5				   0/5		Out

8		   6					  6				   0/6		Out

9		   7					  7				   0/7		Out

10					   7				  6		   1/6		In

11					   8				  7		   1/7		In

12					   6				  5		   1/5		In

13					   5				  4		   1/4		In

14		   9		   1				  1		   2/1		Both

15					   4				  3		   1/3		In

16		  10		   2				  2		   2/2		Both

17		  11		   3						   2/3		Both

-----------------------------------------------------------------------------

Here's the normal pin functions for a printer port.

Pin		Function at PC					Pin		Function at PC

-----------------------------------------------------------------------------

1		Strobe out						14		Auto LF out

2		D0 out							15		Fault in

3		D1 out							16		Init out

4		D2 out							17		Select out

5		D3 out							18		Gnd

6		D4 out							19		Gnd

7		D5 out							20		Gnd

8		D6 out							21		Gnd

9		D7 out							22		Gnd

10		ACK in							23		Gnd

11		Busy in							24		Gnd

12		Paper End in					25		Gnd

13		Select in

-----------------------------------------------------------------------------