Tecmar's Clock Utilities;

**** The Tecmar's clock on our multifunction boards usually is

addressed by the LPT port. LPT1 will represent TIME 1, LPT2

will represent TIME 2. To disable LPT ports will disable the

clock.

TIMTEST;

Program to test the clock.....

TIME 1 (LPT 1) .......  TIMTEST -A1

TIME 2 (LPT 2) .......  TIMTEST -A2

SETTIME;

Program to reset the time and date....

First use the dos time and date commands to input

the correct variables. Then execute the Settime

command, SETTIME for TIME 1 or SETTIME -2 for TIME 2.

exp:

time 14:23 <cr>

date 12/25/91 <cr>

settime <cr>

DOSTIME:

Program to read the Tecmar's clock and put it into the system.

It is recommended that this program be put in the autoexec.bat

file that it is executed on boot-up.

DOSTIME  for lpt 1.

DOSTIME -2 for lpt 2.

############################################################################################################################